Tsunami Pro

User’s Manual

USER’S MANUAL

For

TsunamiPro

PROFESSIONAL AUDIO EDITING SYSTEM

Black Diamond Sound Systems

P. O. Box 140

Glenhaven, CA. 95443

http://www.soundlabs.com

Copyright © 1996-2003

All Rights Reserved

TABLE OF CONTENTS

Introduction	5

Program Overview	5

Cool Things You Can Do	6

Registration	6

System Requirements	6

Customer Support	7

Update Policy	7

Installation	7

Installing TsunamiPro	7

How To Avoid Reading This Manual	7

Sound And Sampling	8

Sound And The Auditory System	8

Sampling Rate	8

Resolution	9

Storage Requirements	9

The File Menu	10

Open File	10

Creating a New Waveform	11

Copying Data To A Newly Created Waveform	11

Saving a Waveform	12

Save	12

Save As (New Format or New Name)	12

Save (Highlighted Area)	13

Saving a File in ADPCM Compressed Format	13

Saving a File in MPEG Layer 3 (.mp3)  Compressed Format	13

Saving a File in AVI Format: AVI File Editing	14

Saving a File In Real Media Format	15

Delete file	15

Close	16

Preferences	16

Display colors	17

Time format	18

Options	19

Startup	20

Toolbars	22

Select WAVE I/O Devices	23

Select MIDI I/O Devices	24

Purchase / Upgrade New Components	25

Web Site	25

Exit Program	26

The Edit Menu	26

Undo / Redo	26

Cut	27

Copying Waveform Sections	27

Drag and Drop	27

Copying to the right or left channel of a stereo waveform	28

Automatic reformatting	28

Copying To A Different Waveform	28

Combine Selection With Destination	29

Replace	31

Mix	31

Clear Waveform Selection	32

Trim waveform	32

Crop Waveform	32

Select all	32

Erase Clipboard	32

The Setup Menu	32

Loop playback	33

Setup playback	34

Chase-lock Playback to External Midi-time Code	34

The View Menu	36

Go to beginning	36

Go to end	37

Go to highlight	37

Cue list	37

Multitrack List: Building a Composite Waveform	38

Magnified Plot: Get Rid of Pops and Clicks: Edit Single Samples	39

Minimum zoom	40

Maximum zoom	40

3D Frequency Spectrum	41

2D Frequency Spectrum	42

Display System Devices	43

CD Player	43

Midi Player	44

Triggered: Synchronizing Midi File Playback With the Recorder	44

Midi Keyboard	45

Mixer	47

Updating the display	48

The Operations (Effects) Menu	48

Accelerando: Change Tempo	49

Amplify	50

Change Pitch	51

Compress / Expand Dynamic Range	53

Echo	54

Scale Waveforms With A User-drawn Envelope	55

Four Channel Parametric Equalizer	57

Graphic Equalizer (10 channels)	58

Fading The Waveform Volume	59

Flange / Chorus	60

Filter Options	62

Sinusoidal Modulation (Tremulo)	64

Noise Reduction	65

Reverberation	69

Reverse	70

Quasi-3D Sound Placement	71

Generate Tones and Special Effects	72

Insert Silence	73

The DirectX Menu	74

Recording a Waveform	76

Waveform Format	80

Change / Set Waveform Format	80

Digital Sampler	80

Synchronize Recording or Playback	81

Wave Player: Four-track Recording and Editing	84

Synchronizing Playback With the Recorder: Four Track Operation	85

Playing a Waveform	86

Play Clipboard Contents	86

Editing a Waveform	87

General Editing Operations	87

Crossfade Two Waveforms	88

Using Time Markers	88

Adjusting the Display: Multiple Editing Windows	90

Getting Help	90

Accelerator Keys	91

Preset Options	91

On-Line Tutorial	92

Windows Clipboard	92

Windows ClipboardWindows Clipboard	92

Highlighted Section	92

In Case Of Trouble	94

License Agreement and Warranty	95

INDEX	96

# Introduction

## Program Overview

TsunamiPro is a program for recording, editing, and playing digital audio files. It was designed by a group of successful professional musicians and by scientists trained at MIT in computer science and auditory physiology, and incorporates a number of original algorithms in digital signal processing which provide some powerful capabilities for the analysis and synthesis of music. The system was designed to be as intuitively simple and easy to use as humanly possible, and in addition to the standard features of recording and playback that are required by users interested in creating sound files for Multimedia applications, TsunamiPro incorporates many advanced features of use to professional musicians interested in creating CD quality recordings. A partial list of the many features of TsunamiPro includes:

- Multiple editing windows

- A Compact Disk (CD) player

- A Midi file player (synchronized with Recorder)

- A Midi Keyboard (play any instrument from your keyboard)

- A built-in Mixer

- The ability to import and export files in different formats

- The ability to synchronize recording with an external time source (SMPTE and MIDI sync signal)

- Streamlined manual editing of waveforms (removal of clicks, etc)

- The ability to copy, mix, and paste files in different formats

- The ability to scale waveform amplitude using a user-drawn envelope

- Sophisticated filtering options, including a four channel parametric equalizer

- FFT spectral analysis, and a variety of digital filters

- Rapid tempo change with or without changing pitch

- Compression and expansion of dynamic range

- A variety of noise-reduction methods

- Rapid pitch modulation with or without changing time

- A host of special effects including flange, chorus, echo,  reverberation, and tremulo

- A variety of preset special effects filters

- The ability to rapidly locate, display, and edit specific sections of a recording using time markers

- The ability to customize the layout of controls, the display colors, and other program parameters

- The ability to insert newly recorded sections into a sound file during recording

- The ability to choose between different soundcards resident in the computer

- Noise gating and trimming waveforms

- Cross-fading, panning, and amplitude adjustment

- Cross copying, mixing, and pasting between files with different formats

- Context-sensitive help

- An On-line Tutorial

- DirectX plugin support

- AVI file editing

- Save wave and AVI files in Real Media format, for streaming from the Internet

- Save wave files in .MP3 format, for streaming from the Internet

- Save wave files in JAF format, for streaming from the Internet

- Use the built-in CD Ripper to rip tracks from a CD / DVD directly to disk.

- Open (decode), Play, Edit, and Save (encode) MP3 files

## Cool Things You Can Do

Here is a partial list of some of the cool things you can do with TsunamiPro:

- Record from a microphone, CD player, or Midi player

- Drag-and-drop sections to copy, paste, mix, etc.

- Cross-fade two selections

- Hand tailor your tracks (trim, crop, clear, insert silence, etc)

- Use timemarks to define selections of interest

- Make a cuelist of selections to play on a file

- Build a composite waveform from multiple tracks

- Add special effects (reverse, echo,  reverb, flange, chorus,  tremulo, etc)

- Enhance single instruments (compress or expand dynamic range)

- Change the volume (amplify, normalize, pan, fade, etc)

- Get rid of clicks, pops, and glitches

- Get rid of noise

- Examine frequency components and calibrate instruments

- Enhance different frequency components (4-channel equalizer, filter options)

- Make quasi-3D sound using only two speakers

- Sync two soundcards for 4-track recording and playback

- Change pitch and tempo

- Use the Mixer

- Use the Midi Player

- Use the Wave Player

- Use the CD Player

- Use the Midi Keyboard

- Sync to other devices (sequencer, SMPTE, chase-lock, etc)

- Communicate with an external sampler unit

- Save files in compressed format

- Record, Play, Edit, and Save  MP3 Files

- Record directly from CD to WAV or MP3 files using the built-in CD RIPPER

## Registration

- We urge you to bite the bullet and register your new program, which you can do simply by clicking on the registration button that pops up when you first run the program. This will whiz you off to our Web Site (http://www.soundlabs.com) on the Internet, where you can fill out the Registration  E-Mail form in a matter of seconds. Registration entitles you to Customer Support, Update Notifications, and other useful information.

## System Requirements

- Computer: 386SX, 486 , Pentium (recommended), and color monitor with Windows 95, 98, or NT drivers

- Memory: 2Mbytes bare minimum, recommend 16Mbytes at least

- Hard disk: 3MBytes for programs, the bigger the better for sound files

- Soundcard: TsunamiPro should work with all Windows 95 or MPC compatible soundcards.

- Microsoft Windows 95, 98, Me, 2000,  NT or later

- Pointing device such as a mouse.

## Customer Support

- Our Customer Support telephone number is (970) 389-2714 if you have trouble either installing or using TsunamiPro. Please note that in order to be eligible for support, you must have registered the program. To expedite support, please have the following information handy:

- Your hardware configuration (make of soundcard, amount of RAM, size of hard disk)

- Nature of the problem

## Update Policy

- Registering TsunamiPro gets you on our list for updates, and for one year from the date of purchase, you are entitled to maintenance updates free of charge. Major updates that add substantial new features will also be provided free to those who purchased TsunamiPro during the three months preceding the release of the update, and will be made available to others at a substantially reduced fee.

# Installation

## Installing TsunamiPro

- TsunamiPro is very simple to install. To Install TsunamiPro, you should be running Windows 95, 98, Me, 2000, or NT (ie, a 32-bit Operating System). Use a utility such as WinZip (available for free from the Internet) to unzip, into a temporary working directory on your hard disk, the file zip you received via download.  Then, from the “Start” menu, choose “Run”, “ Setup”. The Setup dialog box will then appear and post various messages telling you what it's up to. We recommend that you let the Setup program install TsunamiPro in the 'C:TsuPro32' directory which it will create by default. It takes a about a minute to de-compress T32.Exe, as well as the demonstration sound file, which is a short stretch from Russ Freeman's album "The Rippington's: Live In LA". The Setup program will create a Program Group which contains the icons for TsunamiPro, PlayCD (the TsunamiPro CD Player), and Update.Wri, a file that contains additional information that became available after this manual went to press. You can then activate any of these programs simply by double-clicking on their icons. The installation procedure does NOT modify your autoexec.bat, config.sys, registry, win.ini, or system.ini files, or any other files for that matter. It is a simple matter to un-install TsunamiPro, therefore, by simply deleting the files in the  directory in which it was installed.

## How To Avoid Reading This Manual

- If you've read this far, congratulations! Most folks, including those at Black Diamond Sound Systems, hate reading manuals and only do it if, for example, they are sitting in a dentist's office waiting for a root canal and that's all there is to read. So, rather than expecting you to read this manual from start to finish before actually trying out your new program, we expect that you will probably consult it only for reference purposes. For that reason, the manual is organized according to the layout of the various menus and controls used in TsunamiPro, followed by procedures such as recording, editing, saving files, etc. Because you must be very smart to begin with (in order to use a computer in the first place, and to have bought TsunamiPro!), you probably don't really need to read the entire manual unless you want to. Instead, after installing the programs on the Setup disks you can immediately start TsunamiPro by double-clicking on its icon in the new program group just created, and use the on-line Help system to navigate around as the mood strikes you.

# Sound And Sampling

- In order to better understand some of the procedures used by TsunamiPro to record and edit sound files, it is helpful to consider some physical aspects of sound and of the human auditory system, and how these relate to the design and implementation of sound recording devices.

## Sound And The Auditory System

- When you move something rapidly in a physical medium such as air or water, some of the kinetic energy is imparted to the surrounding medium and begins to spread outward from the moving object in "waves" (ie., patterns of alternating increased and decreased pressure). The faster you wiggle the object (say, a violin string), the faster the frequency of the corresponding sound waves. The denser the medium, the faster the sound waves travel through it. (Conversely, in the vacuum of outer space, where there is no medium to propagate the sound waves, there is no sound! Not a good place to go to  listen to Mozart.)

- When a sound wave enters your ear and bumps up against your tympanic membrane (ear drum), a truly incredible chain of events is set into motion, so incredible, in fact, that at first thought you might be persuaded to believe that the human auditory system was designed by a team of sanitary engineers who had taken the Wrong Medicine. Briefly, what happens is as follows: the fluctuating sound pressure changes cause your ear drum to start vibrating at the same frequency (ie, the pressure changes are converted back into mechanical motion), and this motion is "geared down" through a series of tiny bones in your middle ear and finally transmitted to a specialized membrane in your inner ear, shaped like a snail, which  flaps up and down, tickling the even tinier hairs on a group of electrically charged "hair cells", partially discharging them and causing nerve action potentials to go dancing up your auditory nerve to your brain. How the brain analyzes this stream of incoming action potentials is a subject of intense interest to a growing body of brain scientists. Of special interest is the fact that a smoothly varying sound source (called an analog signal) is converted into a discrete number of samples per unit time (called a digital signal) before being analyzed by your auditory system, which is perforce a classic example of an "analog to digital converter".

## Sampling Rate

- In recording music to the soundcard in your computer, a similar form of analog to digital conversion takes place in the hardware. Since humans can hear frequencies in the range of 20 to 22,000 cycles per second (Hertz, or Hz), your soundcard must be able to record these frequencies. Another factor imposes an additional constraint on the hardware, however, and that is that in order to recreate the highest frequency present in a signal, the signal must be sampled by the hardware at at least two times that frequency. A simple way to think of why this is so is to consider a simple sine wave, ie, a pure tone that just wiggles up and down at a constant frequency. To define one cycle of the wave, you need to sample it when it has reached the top of it's excursion AND when it has reached the bottom of it's excursion: that is, you need to sample it twice per cycle. So, in order to achieve maximum fidelity in a recorded sound signal, your soundcard should have a maximum Sampling Rate of at least 44 kHz.

## Resolution

- .

- The human auditory system is also remarkable because it can hear (and make subjective discriminations on) sounds that vary over a tremendous range of intensities. The usual way that this is measured is to look at the ratio of two sound pressure levels: the average person can distinguish sound pressure differences over 10 orders of magnitude. This is such a huge number that sound engineers (and audiologists) take (10 times) the logarithm of the ratio of two sound pressures as the unit of measurement of loudness, and call it the "decibel", or "db". Clearly, expecting your hardware to be able to faithfully resolve 10^{10}th different loudness levels is asking a lot. So how many levels is enough for adequate resolution of the different loudness levels in music? The earliest soundcards sort of ducked this question, for the simple reason that the available hardware was barely fast enough to keep up with the incoming data stream. The next generation of hardware chose the minimum resolution to be the same as the size of an 8-bit register, or one "byte". A byte represents 2^{8}th, or 256, possible values. This turns out to be adequate for music that doesn't contain a lot of loud (or soft) transients. The newest generation of hardware is capable of using 2 bytes (or 16 bits) per sample. This represents 2^{16}th, or 65,536 possible values, which is adequate for the majority of musical sounds encountered in the real world.  Note: some of the latest sound cards can digitize samples to 24 or 32 bits accuracy.

## Storage Requirements

- The price you pay for 16 bit resolution and a 44 kHz sampling rate is a relatively huge requirement for data storage. For example, if you are recording stereo, at 16 bits, and 44 kHz, you will need 10.58 million bytes (mega bytes, or MB) of storage for every minute of music that you record. Few folks come equipped with enough computer memory to handle huge amounts of data such as these, although a 2 minute section of stereo music recorded at 8 bits resolution and 22 kHz will require "only" 5.29 MB of storage, which is readily "doable" with the 8 MB of memory common in many MultiMedia PC computers today.

# The File Menu

- Open file

- Create new file

- Save

- Save As

- Save (highlighted area)

- Delete file

- Close

- Preferences

- Purchase / Upgrade

- Rip CD Tracks

- Exit

## Open File

- Open File

- This option brings up a File Open dialog box which lists all of the stored .wav files on the specified data directory. To select a file, simply highlight the file name and either double click on the name with the left mouse button, click on the OK button, or press 'Enter', and the file will then be loaded into the TsunamiPro editing windows. To select a different directory, double click at the top of the directories listing, then select the desired directory, and double click on that item. The default file extension is ".wav",  corresponding to the standard Microsoft PCM (Pulse Code Modulated) digital audio waveform format. TsunamiPro also supports a variety of other file formats, whose extensions are listed in the OpenFile dialog box. To open a file in one of these other formats, simply select the desired format, and TsunamiPro will automatically convert the file into a .wav format file and open it in a new window.

- Play

- This option allows you to audition a .wav file (in either standard PCM format or in compressed ADPCM format) without having to load it into the display first.

- More

- This option displays any descriptive information that has been stored with the file, such as the Artist, Engineer, Copyright, and other information.

- The attributes of the file are also displayed at the bottom of the dialogue box, including the length of the file, the format, and the type of file (MS PCM, or compressed ADPCM, etc).

## Creating a New Waveform

- There are several ways to create a new waveform. The simplest is to click on the 'Create" button on the Files Toolbar, or select the Create New File option from the Files Menu. This will pop up a dialogue box asking you what format parameters to use (8 bits or 16 bits, mono or stereo, and various sample rate settings). The highest fidelity files, which are also the biggest, use 16 bits, stereo, and either 22kHz or 44 kHz. You can also create a new waveform "on the fly" in an empty editing window, simply by copying the contents of the clipboard to it, or even more simply, by dragging the selection with the mouse and dropping it at the desired location.

## Copying Data To A Newly Created Waveform

- This option is very useful for creating individual instrument tracks, and for building up final mix-down tracks. After you have created a new waveform:

- 1) Switch to the window containing the data to be copied

- 2) Highlight the area to be copied, and click on Copy

- 3) Switch back to destination window  and click anywhere on the window

- 4) Click on Paste (Insert)

- See also:

- Copy

- Cut

- Mix

- Paste

- Undo

- Combine selection with destination

- Waveform format

## Saving a Waveform

- There are menu options with which  to save a sound file. to the hard disk.

### Save

- This option is used to save to disk the .wav file in the current editing window. The file will replace the existing disk file having the same name, and will be stored in the same format that it was recorded in (eg, 8 bits, stereo, 22kHz, or whatever). If you want to write the file in a different format, or give it a different name, use the 'Save as' option under the 'Files' menu.

### Save As (New Format or New Name)

- This option is used to name and save to disk the .wav file in the current editing window. If you want to convert the file to some type other than a .wav PCM file (the default), select the desired file type from the Save as type combo box. Simply type in the desired name of the in the File name edit box. The file will automatically be given the extension that you select, and it is not necessary to type that in. If you want to replace (ie, overwrite) an existing file with the present file, simply select the preexisting file name and double click on it. A dialog box will pop up to confirm that you indeed wish to replace the existing file. If you select 'No', then the new file will NOT be saved.

- Note that you can save the file in a different format than it was recorded in, by selecting the desired format  buttons (eg, 8 bits, stereo, 22kHz, or whatever) in the Attributes group box. This provides a valuable means of reducing the size of a sound file., if you are willing to sacrifice some fidelity. For example, if you recorded a sound file. at 44 kHz, stereo, and 16 bits but stored it at 22 kHz, stereo and 8 bits, the file size would be reduced by a factor of 4. You can also do the converse operation, for example, by making a mono file into a stereo file, although in this case both channels will have the same set of samples.

### Save (Highlighted Area)

- This option lets you save the highlighted area of a waveform as a separate sound file. This is a very convenient feature when you want to extract a section of music from a larger file. The newly saved file will have the same format characteristics as the parent file.

TsunamiPro supports saving files in multiple formats. The following describes some special formats of interest.

### Saving a File in ADPCM Compressed Format

- This option compresses the file into Microsoft ADPCM format before saving the file to disk. The file will be automatically de-compressed into the standard PCM format when it is read back from the disk.

### Saving a File in MPEG Layer 3 (.mp3)  Compressed Format

TsunamiPro enables you to convert standard .wav files to MPEG layer 3 (.mp3) format. This is a very popular format that uses a proprietary compression scheme to reduce the file size by about 10-fold, while still preserving near-CD quality sound. The reduced file size and high-quality sound make this a very popular format in use on the Internet. In fact, distribution of .mp3 files from the Internet is seriously threatening the recording industry's sales of CDs. The latest version of the RealMedia G2 Player can play .mp3 files streamed from the Internet Note: TsunamiPro uses a very high-quality proprietary MP3 encoder / decoder that comes with the distribution / setup file.

### Saving a File in AVI Format: AVI File Editing

- TsunamiPro allows you to edit and save AVI (Audio-video interleaved) files. To load in an AVI file, choose the .avi file type on the Open File dialog box and select the desired file. A pictorial representation of the video frames in the AVI file will then appear in the editing window above the sound representation, as shown above. Use the standard Zoom in and Zoom out buttons to change the display. The video frames that are associated with the visible section of the audio waveform will appear above it. You can use all the standard Cut, Copy, Paste, etc. features to edit the audio file, and the spacing of the video frames will be recalculated and displayed automatically. (Note that you cannot presently Cut, Copy, Paste, etc. the video frames.) When you Play the file, the display will automatically whiz to the beginning of the highlighted section (or to the beginning of the file, if there is no highlighted selection) and begin displaying the audio and associated video frames at the left edge of the screen. To see the video frames at maximum resolution, you should set the zoom to Maximized, which you can do by clicking on "Maximum zoom" on the Edit menu. During playback, the display will automatically move across the screen, with the current file position at the left-hand edge of the screen. When you Save the edited file, make sure that you select the .avi file type on the Save File dialog box.

### Saving a File In Real Media Format

- TsunamiPro provides the ability to compress and save audio and .AVI files in RealMedia format that can be streamed in real-time from the Internet, producing near CD-quality sound. It uses the latest codecs from RealNetworks (see http:www.real.com on the Web). To save an audio or AVI file in this format, choose "RealMedia files (*.rm)" from the File types combo box on the Save dialog box. The RealMedia Audio/Video Compression Parameters dialog box, shown above, will then appear, allowing you to choose the desired compression parameters. An abbreviated description of all of the available Audio and Video compression parameters will be displayed in the Audio and Video 'Choose desired type of compression' combo boxes. Pressing 'OK' will start the file conversion, and the Progress bar at the bottom of the dialog box will indicate the progress of the compression. Pressing "Cancel" at any time will stop the compression and return you to TsunamiPro. The files will be saved with an '.RM' extension, and can be incorporated into your Web page and uploaded to your web site, or to your server if it supports streaming RealMedia files.

## Delete file

- As its name suggests, this option is used to delete a file from the hard disk. This is sometimes useful, since you can manage your list of .wav files without ever having to leave the Windows environment. Simply highlight the name of the file that you wish to delete, and either double click on  the name, or press OK, and presto, the file is history...

## Close

- This option deletes the contents of the current editing window, and frees up any memory associated with it.

## Preferences

- This option allows you to customize various aspects of TsunamiPro's operation. Please see the individual help topics for each of the Preferences tabbed dialog box.

- Colors

- Time format

- Options

- Startup

- Toolbars

- Wave Devices

- Midi Devices

- All of the options listed on the Preferences dialog box are saved when you exit TsunamiPro, and will go into effect the next time you start the program. You can change these options as often as you like during an editing session.

### Display colors

- This option lets you customize the colors with which TsunamiPro will display the waveform being edited. Windows provides 16 'pure' colors, as shown in the 16 colored squares on the Customize Display Colors dialog box. (More colors than this require either the use of 'dithering', which looks lousy on some monitors, or the use of customized color palettes, which can louse up the appearance of concurrently running applications.)

- To change the display colors, you have six options. Clicking on the 'Waveform' button will select the color of the actual waveform. Simply click on the desired color (a small rectangle will appear in the center of the selected color, indicating it has been chosen as the target color).  Similarly, you can change the window background, the color of the grid, and the color of the waveform peaks, and the colors of the time markers and the time marker text by clicking on the 'Background', 'Grid, 'Wavecrest', 'Marker Color' and the 'Marker Text'  buttons. (Note that you can make the grid go away if you want to by setting it the same color as the background.) Clicking on the 'Save' button will save the new color scheme, which will then be used the next time you start TsunamiPro. Clicking on the 'Default' button invokes a default color scheme, chosen to be not too bilious by the majority of non-colorblind users consulted...

### Time format

- You can set the format in which time is displayed using one of the five options listed under 'Time format'.  'SMPTE'  (Society of Motion Picture and Television Engineers) time formats, commonly used in the motion picture and music industry, display time in hours, minutes, seconds, and frames per second.

### Options

- Zoomed startup display

- This option loads in selected waveforms at maximum zoom, which somewhat speeds up loading in a file.

- Enable undo operation

- This option enables / disables the Undo / Redo feature, with a marginal increase in speed. Default is checked.

- Voice message

- This option plays a "Ready to go" voice message when editing operations are complete.

- Big time display

- This option displays a big elapsed-time indicator during waveform playback that is easily visible at a distance.

- Enter marker while playing

- This option pops up a small dialog box each time you enter a timemark (by pressing 'm' during waveform playback) into which you can enter a desired name for the timemark, rather than by right-clicking on the timemark after it has been created and typing in the name then.

- Show tip of the day

- This option turns on or off the "tip of the day" dialog box that pops up when TsunamiPro is first booted up.

### Startup

- Display on startup

- This enables or disables the display of the TsunamiPro logo on program startup.

- Display now

- This displays the TsunamiPro logo when clicked. To make the logo go away, simply click on it.

- Data directory

- This option allows you to set a new data directory where the .tmp files generated by TsunamiPro during its various operations are stored. This enables you to choose a disk drive that has adequate free space.

- TsunamiPro performs much of its intermediate editing work on files which it creates and gives the extension '.tmp' to. (These files are then deleted when they are no longer required.) Many other programs use a similar strategy. The question is, where should these temporary files be located? Most programs, including Windows itself,  use the '\temp' directory that is usually specified in your autoexec.bat file. Oftentimes, however, users wish to set their '\temp' directory to a ram disk, for speed. This is cool provided you have lots of memory; otherwise, you can easily run out of memory on the ram disk when in the midst of editing a big file. Thus, the designers of TsunamiPro have provided you the flexibility of letting you store your .tmp files wherever you want to. Most often, you need not concern yourself with where your .tmp files are: TsunamiPro will put them on the same directory as your .wav files are stored, by default.

### Toolbars

- Toolbars and Tooltips

- Floating and Docked Toolbars

- TsunamiPro provides a very flexible way to configure the various controls. There are ten separate toolbars, each controlling a different logical set of functions. Each of these toolbars can either be hidden (by clicking off the corresponding check box), floating, or docked (ie, inserted into the main window border at any one of its four edges). To float a hidden toolbar, simply click on the corresponding check box. You can then drag the toolbar wherever you want by holding the left mouse button down in the caption area. You can also change the shape of the toolbar by dragging on the size controls on its borders. To dock the toolbar, move it to the desired location along one of the sides of the main window. When the toolbar is in a dockable zone, its outlined shape will change to the shape and position it will occupy if you let go of the mouse button. To refloat a docked toolbar, simply position the mouse cursor in the margin of the toolbar, press the left button, and drag it away.

- Default setup

- This option docks all of the toolbars in a default arrangement.

- Tooltips

- This option enables the display of a short identifying message as you hold the mouse cursor over a control for a brief period.

- Big volume indicators

- This option increases / decreases the size of the LED volume indicator and the DB Meters on the main display.

Select WAVE I/O Devices

- Most soundcards contain the electronic circuitry for recording and playing back digital audio (which is stored on your hard disk in the form of ".WAV" files), as well as the circuitry for recording and playing back MIDI data. (MIDI, which stands for "musical instrument digital interface", consists of a series of coded electronic signals which can be used to synthesize the sound of different musical instruments, rather than the signals made by the actual instruments themselves. Midi data is stored on your hard disk in the form of ".MID" files.) Windows programs such as TsunamiPro communicate with the electronic circuitry on your soundboard via software interfaces called "drivers", which in turn are supplied by the soundboard manufacturer. If you have more than one soundboard installed on your computer (or more than one set of drivers for a given soundboard), then any well mannered program should provide you with the ability to choose which driver (and thus, which soundboard) to use in recording or playing back either digital audio or midi data.

- This option lets you select which digital audio recording and playback devices to use as input and as output in systems that have more than one soundcard installed. Simply select the desired device name from the list of soundcard input and output devices displayed in the relevant combo boxes.

### Select MIDI I/O Devices

- Most soundcards contain the electronic circuitry for recording and playing back digital audio (which is stored on your hard disk in the form of ".WAV" files), as well as the circuitry for recording and playing back MIDI data. (MIDI, which stands for "musical instrument digital interface", consists of a series of coded electronic signals which can be used to synthesize the sound of different musical instruments, rather than the signals made by the actual instruments themselves. Midi data is stored on your hard disk in the form of ".MID" files.) Windows programs such as TsunamiPro communicate with the electronic circuitry on your soundboard via software interfaces called "drivers", which in turn are supplied by the soundboard manufacturer. If you have more than one soundboard installed on your computer (or more than one set of drivers for a given soundboard), then any well mannered program should provide you with the ability to choose which driver (and thus, which soundboard) to use in recording or playing back either digital audio or midi data.

- This option lets you select which MIDI recording and playback devices to use as input and as output in systems that have more than one soundcard installed. Simply select the desired device name from the list of soundcard input and output devices displayed in the relevant combo boxes.

## Purchase / Upgrade New Components

### Web Site

- This option activates the built-in TsunamiPro Web Browser, which in turn dials up your Internet Provider and takes you to the Black Diamond Sound Systems Home Page on the World Wide Web (blackdiamondsound.com), where you can order an upgrade through a Secure On-line Ordering System.

- Rip CD Tracks

TsunamiPro comes with a built-in "CD Ripper", with which you can "rip" selected tracks from a CD-ROM or DVD drive and record them directly to disk, bypassing your soundcard in the process.

Available CD Drives:

Use the combo box scroll control to view the CD / DVD drives installed on your computer. Highlight the desired drive.

Tracks on CD (Select one to rip or click all)

You can rip one track at a time, or (by selecting All Tracks), all of the tracks on the CD in one operation

.

Rename selected track if desired...

The contents of the listbox show the tracks on the currently selected drive (see above). To rename a track, select the track to be renamed from the listbox, then type in the new name, and click the "Rename" button. The new name should then appear in the listbox.

Select Output File Type:

TsunamiPro can rip tracks in three popular formats: 1) standard PCM .WAV files; 2) compressed .MP3 files (320 kb/s); and 3) PCM .RAW files (no header). Simply click on the desired option. You can also change the directory where the ripped files will be stored. Simply select the desired directory from the "Select Output Directory" listbox, and the desired drive from the "Drive" listbox.

- Press "OK" to start ripping a track, and "Cancel" to abort ripping a track. The progress of the operation will be displayed in the progress bar at the bottom of the dialog box, as well as the time remaining before completion.

## Exit Program

- This option will terminate any playback in progress, perform all the necessary chores for politely cleaning up after itself, and exit TsunamiPro back to Windows.

# The Edit Menu

- Undo / Redo

- Cut

- Copy

- Paste

- Replace

- Mix

- Clear

- Crop

- Select all

- Erase Clipboard

- Play Clipboard Contents

## Undo / Redo

- Undo

- If, after playing a section of a waveform most recently modified by an editing operation (such as Cut, Paste, Flange, etc), you do not like the results of the operation, you can undo it (ie, restore the previous waveform condition), by simply clicking on the Undo control. This will restore the previous version of the waveform and redraw the display. The most recent edit operation is indicated in the topmost item in the Edit menu, in case you get distracted (for example, by the pussy-cat chewing through the cable to your earphones, a not infrequent event at the black Diamond Sound Systems).

- Note that you can also disable the undo feature, by turning off the 'Enable undo operation' button on the Options tab of the Preferences dialog box, accessed from the Files menu. This will save a fair amount of time when editing large stretches of a big waveform, but it is not without risk unless you have already backed up the file.

- Redo

- After undoing an operation, you can instantaneously redo it  by clicking on the Redo button. The Undo / Redo operations can be repeated as many times as desired. This makes it very convenient in deciding whether you really like the results of the most recent editing operation.

- See also:

- Copy

- Cut

- Mix

- Paste

- Create New File

- Preferences

## Cut

- This option is used to remove a section of a waveform (or an entire waveform). First, select the section to be removed (ie select the highlighted section). Then, simply click on the Cut control. This transfers the section to the clipboard (so that it  can be used to copy, paste, or mix to another location, or to restore to the original), and also deletes the section from the waveform and redraws the display.

- See also:

- Copy

- Paste

- Replace

- Mix

- Undo

- Create New  File

- Combine selection with destination

## Copying Waveform Sections

### Drag and Drop

- By far the easiest means to copy selections between files is to use drag and drop. To do this, highlight the desired selection to be copied, then click on the highlighted selection with the left mouse button, and while holding the button down, drag the selection to the desired location. This can be to another window, to a blank area of the main window (which will automatically create a new file), or to the files cuelist. To copy a selection to an existing (destination) window, first activate the destination file by clicking on it (or selecting it in the Window menu), and position the caret where you want to insert the section to be copied. Then activate the source window, highlight the area you want to copy, and move the mouse cursor over the highlighted area with the left button held down. The cursor will change to a copy-folder cursor. While still holding the left button down, drag the selection to the destination window and release the left button. This will cause the Combine Selection With Destination dialog box to pop up, from which you can select how you want the selection to be included in the destination waveform (ie, inserted, replace, mix, or crossfade). The selection is then automatically copied to the destination waveform, and is automatically reformatted in the process. As mentioned above, you can use the same method to add selections to the cuelist and to the multitrack list.

### Copying to the right or left channel of a stereo waveform

- TsunamiPro allows you to copy FROM either the right or left channel (or both) of a stereo waveform by simply highlighting the desired selection. When the cursor is in the top one-fourth of the viewing window it changes to a "L" cursor, indicating that whatever you highlight will refer to the left channel (which always occupies the upper track). Similarly, when the cursor is in the bottom one-fourth of the view window it changes to an "R" cursor, and will highlight only the right channel, which always occupies the bottom track. To copy TO a single channel of a stereo waveform, select either the Left channel or Right channel radio button under the Mix option, and the source selection will replace either the left or the right channel of the destination waveform selection. (With this option, the fader mixer controls are ignored.)

### Automatic reformatting

- A powerful feature of the Copy operation is that you can copy samples between two sound files that are recorded in different formats, and TsunamiPro will automatically convert the samples into the appropriate format (stereo vs mono, 8 bit vs 16 bit), and the sample rates of the two files need not be the same.

- Using the Edit Menu or the Edit Toolbar Controls to Copy Waveform Selections

- It is very easy to copy sections of a waveform (or an entire waveform) from one region to another within the same waveform, or between different waveforms. This operation makes use of the  standard Windows Cut / Copy / Paste operations which employ the Windows clipboard, which in turn is a block of memory that temporarily stores data to be transferred. As an example , suppose you want to copy a drum roll  near the start of a waveform to a place near the end of the waveform. You would simply highlight the drum roll, and click on Copy. Then move the cursor to the place on the display where you want to copy the  drum roll to, and click Paste (Insert), and presto, the section is copied.

- What happened here? When you click on Copy, the drum roll section is magically copied to the clipboard (either to a block of memory managed by Windows, or to a disk-file managed by TsunamiPro).  When you click on Paste, a copy of the clipboard contents are actually inserted at the new location. The original data is still available in the clipboard, and is not removed from the clipboard until you invoke the Erase Clipboard option from the Edit menu, or when you copy something else to the clipboard.

- Note that when the data was actually  copied to the new location, the Undo button becomes enabled, and the  "Undo Paste" appears Edit menu. This signifies  that if, after playing the waveform, you do not like the results of the copy operation, you can undo it (ie, restore the  previous waveform condition). In general,  clicking on the Undo button when it is enabled will erase the most recent edit-operation, whereas clicking on the Redo button will instantly redo the previous operation.

### Copying To A Different Waveform

- Copying a section of a waveform to a different waveform is exactly  analogous to the steps just described. First, highlight the section to be copied and transfer it to the clipboard by clicking on Copy. Next,  activate the destination waveform (ie, click on the Window  that contains the destination waveform) and then  click on its display window at the point where you want to copy to. Then,  click Paste on the destination waveform. Note: the destination waveform can either be a preexisting waveform read from disk,  a newly recorded waveform, or a newly created file.

- See also:

- Undo / Redo

- Cut

- Paste

- Replace

- Mix

- Create new file

- Combine Selection With Destination

### Combine Selection With Destination

- This dialogue box lets you specify how you want the selection included in the destination waveform.

#### a)	Insert

- The selection will be inserted at the position of the caret, thereby lengthening the waveform. The original data will be copied at the end of the inserted data.

#### b)	Overwrite

- The selection will replace the equivalent number of samples in the destination waveform, which will be lost.

#### c)	Paste

- If the destination has a highlighted selection, it will be replaced with the source selection.

#### d)	Mix

- The source and destination data values will be mixed together in the proportions set by the Source and Dest fader controls.

#### e)	Crossfade

- If the destination file has a highlighted selection, the source selection will be added to the destination waveform beginning at the start of the destination selection, and the destination selection will be faded out while the source selection is faded in over this interval. The initial amplitudes of the source and destination data is set with the corresponding fader controls. The fading can either be linear, or exponential (which compensates for the actual sound energy in the two waveforms).

- Crossfade Example:

- As a practical example, assume you have two files, named "A" and "B", and you want a new file that starts with file A, then gradually crossfades into file B. First click on file A and highlight the end of the waveform for a length of time equal to the length of time you want the crossfade to take, say 10 seconds. Then click on file B and select "Select All" from the Edit menu. Place the cursor somewhere in the highlighted region of file B and drag it to a position over file A, and then release the left mouse button. Select "Crossfade" from the Combine Selection With Destination dialog box. TsunamiPro will then extend file A such that its original tail end will fade into file B.

#### f)	Paste

- This option is used to transfer a section of a waveform (or an entire waveform) to a new location, which can be either in the same waveform or in another waveform. First, select the section to be pasted (ie, select the highlighted section) and click on the Copy (or the Cut) control. This transfers the section to the clipboard, where it becomes available for copying, cutting, pasting, and mixing. There are three methods of pasting the data.

- 1) Insert samples at specified position, preserve original data

- To insert the clipboard samples, simply click on the destination waveform at the desired position. The Paste control label will say "Insert". Click on Paste and the samples will be added to your waveform, thereby lengthening it.

- 2) Remove samples in highlighted area and replace it with clipboard contents

- Highlight the samples to be replaced. The Paste control label will say "Paste". Click on Paste and the old samples will be removed and the samples data inserted at the start of the highlighted area, thereby changing the length of the file.

- 3) Replace the samples with the clipboard contents, preserve file length

- Click on the destination waveform at the desired position, then click on the Replace control. The clipboard contents will replace the samples in the destination waveform.

- Cross Pasting

- A powerful feature of the Paste operation is that you can paste samples between two sound files that are recorded in different formats, and TsunamiPro will automatically convert the samples into the appropriate format (stereo vs mono, 8 bit vs 16 bit), and the sample rates of the two files need not be the same.

- See also:

- Replace

- Copy

- Cut

- Mix

- Undo

- Create New File

- Combine selection with destination

## Replace

- This option replaces a section of a waveform with the contents of the clipboard, without changing the length of the waveform.

## Mix

- This option is used to mix a section of a waveform (or an entire waveform) with the waveform samples at a different location, which can be either in the same waveform or in another waveform. First, select the section to be mixed (ie, select the highlighted section) and then click on the Copy control. This transfers the section to the clipboard, where it becomes available for copying, cutting, pasting, and mixing. Next, click on the display window at the position where you want to mix to. Then, simply click on the Mix control, and the highlighted section will be mixed with the waveform samples at this new location. The program will ask you how you want to mix the two waveform sections. The mixed samples will consist of the weighted average of the sum of  the "source" samples (ie, the samples in the clipboard) and the "destination" samples (ie, the samples in the waveform segment that you are mixing to). The "% Mixing" control determines the relative weighting factors for the source and destination samples. For example, 50% mixing  results in equal weighting, and the mix will be the simple average of the source and destination samples. If you specify 100% mixing, the output samples will consist of 100% "source" samples (and 0% "destination" samples), whereas if you specify 0% mixing, the output samples will consist of 0% "source" samples (and 100%"destination" samples).

- Cross Mixing

- A powerful feature of the Mixing operation is that you can mix samples between two sound files that are recorded in different formats, and TsunamiPro will automatically convert the samples into the appropriate format (stereo vs mono, 8 bit vs 16 bit), and the sample rates of the two files need not be the same.

- Note that the Mix procedure, combined with other editing options such as Amplify, Fade, etc, allow you complete flexibility in creating new tracks.

- See also:

- Copy

- Cut

- Paste

- Undo

- Create New File

- Combine selection with destination

## Clear Waveform Selection

- This option erases the highlighted section, ie, it sets all of the data sample to zero, without changing the length of the waveform, and without saving the selection in the clipboard.

## Trim waveform

- With the 'Trim' button, all waveform data points that fall outside of the highlighted selection are set to zero. This is useful for trimming off the noise at the start and at the end of a recording, which can be done in one fell swoop at the click of a button, by simply highlighting the music that you want to hear. Everything not highlighted goes away.

## Crop Waveform

- This options lops off the non-highlighted section of a waveform (ie, both ends), shortening the length by the amount lopped off.

## Select all

- This option is used to select (ie, highlight) the entire waveform, even if a portion of the waveform is not visible in the current viewing window.

## Erase Clipboard

- This option clears out the clipboard contents, thus freeing up memory.

# The Setup Menu

- Loop playback

- Setup playback

- SMPTE Chase-lock

- Sampler

## Loop playback

- This option allows you to set the number of times you want a selection to play. You can have it play continuously, or only a specified number of times. You can also set the time for triggered playback to begin using the associated controls.

## Setup playback

- This option lets you play selected portions of a sound file repetitively. You can set TsunamiPro to play a highlighted selection continuously with the Continuous option , or a specified number of times, either beginning immediately (with the Loop count option), or after a lead-in delay, with the Trigger playback on MTC option. This makes it very easy to create repeats and to duplicate measures when recording the output of your soundcard to an external recording device.

## Chase-lock Playback to External Midi-time Code

- SMPTE Chase Lock

- This option is used to lock playback or recording to an external SMPTE track, thus allowing TsunamiPro to be synchronized to an external device such as a tape recorder or VCR. It requires the appropriate hardware, such as an MQX32M card (Music Quest) or a device that can put out Midi Time Code (MTC). Once TsunamiPro is locked onto the external SMPTE signal, you can stop, rewind, or fast-forward the tape recorder, and TsunamiPro will resume playback at the new location. This makes it extremely easy to record, edit, and replace sound at specific locations on an external recording device. This is accomplished by the following steps:

- SMPTE Set Time

- Use the indicated controls to set the desired SMPTE frame count (eg, 24, 25, 30 or 30D frames per second), and the desired starting frame (on the external tape recorder or other device) at which to begin playback or recording.

- Chase-lock to SMPTE dialog box

- This dialog box pops up after you have chosen the starting frame to lock to. . Pressing "Start" arms TsunamiPro to begin playback or recording when the specified SMPTE frame is received from the external device. At this point , start the external device, and TsunamiPro will display the incoming SMPTE numbers in the Synchronization Dialog box at the bottom right of the screen. You can fine-tune the synchronization plus or minus one frame at a time by using the "Adjust sync" arrow buttons on this dialog box.

- Communicating With An External Sampler

- This feature is supplied with the optional Sampler Module

# The View Menu

- Go to beginning

- Go to end

- Go to highlight

- Cue list

- Magnified image

- Minimum zoom

- Maximum zoom

- Frequency Spectrum

- Display System Devices

- Display CD Player

- Display Midi Player

- Display Midi Keyboard

- Display Mixer

- Update Display

## Go to beginning

- This option moves the display / caret to the beginning of the active waveform.

## Go to end

- This option moves the display / caret to the end of the active waveform.

## Go to highlight

- This option moves the display so that the highlighted selection is visible in the center of the display.

## Cue list

- This option lets you set up a group of selections within a sound file. Each selection is identified by a user-specified name and description, and can be played independently. Thus, for example, you can tag individual sections of a long wave file, such as instrumental solos, choruses, or measures, and can play them to an external recording device in any sequence. In combination with the Set Playback Loop feature, each member of a cue list can be repeated a desired number of times. This makes it very easy to create a complex composition from a given sound file.

- The easiest way to make a new entry into the cuelist for a given waveform is simply to drag the highlighted selection onto the cuelist and drop it there by releasing the left mouse button.

- Note that you can dock the cuelist into the main windows left border. The button with the Blue Flag on it activates the Multi-track dialog box, with which you can build a composite waveform consisting on many different tracks, in any format.

### Multitrack List: Building a Composite Waveform

- The Multitrack list provides a very convenient way to organize a group of selections from the same or from different files into a single group that will be played in sequence, or to make a composite waveform from a group of selections. (Press the drop-down control on the Cuelist, then highlight the desired timemark, then drag the highlighted selection from the combo-box edit control.)

- Adding Items

- To add items to the Multitrack list, simply drag the desired selection from the Cuelist associated with the desired file, and drop it onto the Multitrack list by letting go of the left mouse button.

- Deleting Items

- Highlight the entry to be deleted and click on the Delete button.

Play

- This will play each selection on the Multitrack List in sequence, starting at the highlighted entry on the list. To stop playing, simply click the Play button again.

- Building a Composite File

- As above, highlight the first entry to be included in the new file, and click on the Build button. This entry and all that follow it will then be added to a new composite file. Select the desired format of the new file from the Format dialogue that pops up. All of the entries in the Multitrack list will be automatically reformatted to the desired format as they are included in the new file.

- See also:

- Cue list

- Timemarks

## Magnified Plot: Get Rid of Pops and Clicks: Edit Single Samples

- The "Magnified image" option on the View menu brings up the Magnified Display dialog box, which, as its name suggests, contains a magnified plot of the waveform currently being edited. The plot starts at the beginning of the highlighted section, if there is one, or at the beginning of the waveform, if there is not. The magnification is set so that every individual data point is visible. As in the regular editing screen, the left channel is on the top, and the right channel is on the bottom, if this is a stereo waveform. A small drawing tool appears whenever the cursor is inside of either window. You can use this tool to manually change the waveform data points, simply by holding down on the left button. By moving fairly slowly, you can change every data point under the drawing tool to a new position at the tip of the tool. The horizontal scroll bar allows you to navigate from one end of the selected section to the other. The time at the beginning of the display is shown in red at the right side of the scroll bar.

Moving the display

You can move the magnified area under view by sliding the scroll-bar. The Left and Right buttons on the scroll-bar move the display forwards or backwards in time by 1 msec.

De-Glitch: manual removal

TsunamiPro can automatically eliminate unwanted clicks and pops that might be present in the waveform. Such artifacts are usually  represented by sudden transitions in the waveform data, and usually saturate (ie, go to full positive or negative values). They can easily be eliminated manually by simply drawing through them with a smooth transition, or, you can make up some wiggles if the mood strikes you.

De-glitch: automatic removal

Use the Setup dialog box to specify what kind of events to detect. These can be a glitch (ie, a rapid transient), a DC level, or a stretch of silence.  The Threshold slider in the Set Search Parameters dialog box sets the threshold for the event(s) to be detected. The range is 0-100% of the maximum possible value in the waveform (255 for an 8-bit waveform, 65535 for a 16-bit waveform).

The Find next glitch button in the Magnified Display dialog box automatically advances to the next occurrence of the selected event and removes it, whereas the De-glitch all button starts at the current position of the horizontal slider and finds and removes all glitches from there to the end of the selected section.

See also: Noise Reduction (The FFT Noise Removal Feature is a great way to get rid of pops and clicks!)

## Minimum zoom

- This changes the Zoom factor to the minimum value.

## Maximum zoom

- This changes the Zoom factor to the maximum value.

## 3D Frequency Spectrum

- This option employs Fast Fourier Transform (FFT) analysis to compute the frequency components present in that portion of a waveform outlined by the highlighted section (or the whole waveform, if selected). The results are displayed as a quasi-3-dimensional plot, which to some might appear as something out of a Hieronymous Bosch moonscape, and to others might reveal the subtle aspects of how the frequency components vary with time. This knowledge is quite useful for selecting optimal parameters to use for digitally filtering waveforms, and for revealing the effects of digital filtering.

- The basic operations of the frequency analysis are as follows. First, the selected segment is broken up into a fixed number of time epochs. The number of such epochs is set using the 'Disp' slider control, and can range from 4 to 64. For each epoch, a number of consecutive samples of the waveform, sufficient to recover the maximum amount of frequency information, are used to compute the frequency spectrum. Starting at the front of the plot, the frequency  is plotted on the X axis, and the amount of energy associated with each frequency value is plotted on the Y axis. The time noting the beginning of the epoch is plotted on the right hand side of the plot. The same procedure is then repeated for the next epoch, which is plotted slightly above and to the right of the preceding spectrum, and so on, until all epochs have been plotted. It is a simple matter to compare the amount of energy at a particular frequency in the selected segment as a function of time, by simply picking a point at the front of the plot (say, 1000 Hz), and looking along all of the individual plots at the same relative time. You can change the vertical scale using the 'Gain' slider. The 'Update' button will redraw the display.

- To look at the frequency components in greater detail, you can use the 'Browse' feature, which brings up the 2D Frequency Spectrum Display dialog box.

2D Frequency Spectrum

- The 2D Frequency Spectrum dialog box lets you 'browse' through an entire selected section of a waveform (or the entire waveform, for that matter), and examine the frequency components present in each short stretch of samples. The number of samples is determined by the sample rate of the waveform. In order to recover ALL of the frequency components present in a set of samples, a fundamental law of sampling theory (the Nyquist sampling theorem) states that you have to sample at 2 times the maximum frequency. Since you can't change the rate at which the waveform has been sampled once its been sampled, this means that the most you can squeeze out of the analysis is one-half of the sampling rate, which is displayed at the bottom of the plot on the X axis. You can change the horizontal scale factor either by a factor of 2 (by activating the 'X Mag x2' button), or display the horizontal axis in a logarithmic scale (by activating the 'Log scale' button). You can change the vertical scale by using the 'Vert scale' slider control. To browse through the selected section of the waveform, you can use the 'Next' button to show the frequency components in the next consecutive group of samples, or the 'Prev' button to show the frequency components in the previous consecutive group of samples. Holding down 'Next' or 'Prev' will repetitively move the analysis and display. The beginning time of the analysis epoch is shown at the top of the plot.

- See also: 3D Frequency Spectrum

## Display System Devices

- This option displays the characteristics of the software drivers for all of the sound-related hardware devices that are currently installed in your system and recognized by Windows. In addition to displaying the name of the software driver, the current version of the driver is displayed. This information becomes important when you need to decide whether to upgrade your system to a different driver.

CD Player

- The TsunamiPro Compact Disk (CD) Player is activated when you press the "CD player" button on the main edit screen, or when you press the 'Show CD' button on the Record Waveform dialog box. The CD Player will arrange itself so that only the first 10 tracks on the CD that is currently in the CD caddy are visible. To display more tracks, simply press the 'Tracks' button and the next bank of 10 tracks will be listed, if they exist. (If the CD has fewer than 10 tracks, the Track button will not be displayed.) To start play, simply click on the number of the track that you want to play, or click on the "Loop" button, and play will start from the most recently selected track, and will continue playing sequential tracks. Pressing the left or right arrow buttons moves playback to the preceding or to the next track, respectively. Pause pauses, Stop stops, Eject ejects (if your CD player has this capability), and Power (upper left hand button with green bar) closes the CD player. The length of the selected track and the relative position of playback are shown at the bottom of the dialog box. You can whiz to a new section of a track by using the scroll bar: simply click on the left or right arrow buttons to move it backwards or forwards, or hold down the button to move to a new position on the track.

Midi Player

- The Midi Player is equipped with a volume control, a time display (minutes:seconds, updated once per second) and a visual display of active midi channels. The latter provides a visual display of musical notes that are played on each of the 16 different possible midi channels. Each vertical triplet of colored rectangles corresponding to a given channel lights up for the duration of the notes in any track that are played on that channel. The Midi Player also allows you to play a midi file in synchrony with recording and playback of wave files on the Recorder. Thus, you can record an accompaniment to a midi file while it plays.

- Description Of Controls

- File

- This control activates a standard OpenFile dialog box with which to select the desired Midi file.

- Info

- This control opens the "Select Midi Events To View" dialog box, which in turn displays detailed information about the selected midi file.

### Triggered: Synchronizing Midi File Playback With the Recorder

- Once you have loaded in a midi file, this control displays a drop-down dialog box, and sets the Midi Player in "Triggered" mode. In Triggered Mode, when you press Record on the Record Dialog Box, the selected midi file will play for the specified lead-in delay (described below), and then recording will automatically start and continue for the length of time you have selected on the blue bar. To select this time, simply hold the left mouse button down and drag it to the desired position. Both recording of the wave file and playback of the midi file will stop automatically at the end of the highlighted selection on

- the blue bar. When you press Play on the Midi Player in Triggered Mode, the midi file will play for the specified lead-in delay, and then the newly recorded wave file will begin playing. Each small red bar on the horizontal blue bar represents a single measure of music in the midi file.

- Options

- This control brings up the "Go To Bar" dialog box, which contains Start and Stop controls with which to select the beginning and ending bar (measure) of music in the midi file to play. Thus, for example, if you want to record a guitar accompaniment to music in a mid file that starts at the 20th bar, and that continues for four bars, simply set Start = 20 and Stop = 24.

- Lead-in Delay

- With this control you can set a "lead-in delay" with the spin controls, which will start the midi file playing by the specified duration BEFORE actual recording begins on the Recorder. This enables you, for example, to get up to speed if you are recording a live accompaniment.

- Adjust sync

- Because of the multi-tasking nature of Windows, the playback of the recorded wave file and the selected midi file may not initially be in perfect synchronization. You can adjust the synchronization with the Adjust Sync scroll bar. Each click on the scroll bar arrows will be recorded and will go into effect the next time that you press Play on the Recorder.

- Start / Stop / Length

- If you are not sure of which bars  you want to play in the midi file (see "Options", above), you can preview the desired section to be played by using the mouse to highlight a selection. The Start time, Stop time, and Length (duration) of the highlighted section are displayed in these controls in SMPTE-24 format (hours:minutes:seconds:frames).

## Midi Keyboard

- This option uses the internal synthesizer built into your soundcard to generate MIDI notes through the Midi Out Port of your soundcard, which in turn can be used to trigger Playback or Recording on the Recorder. (Note that the Midi In port must be connected to the Midi Out Port of your soundcard in order to use the Midi Keyboard to trigger Recording or Playback of the Recorder.) The Default keyboard is centered on middle C ("C5") and uses a piano voice. The "Wide" check box allows you  to select a different sized keyboard, and the "Label Keys" check box to number the keys. The "Setup" option allows you to configure what midi input and output ports to use on your soundcard, as well as change the instrument, and the midi channel.

- Your computer keyboard is a piano keyboard

- The "qwerty" row of keys and the top row of keys act just like a piano keyboard. You can play multiple notes simultaneously!

- You can also use an external keyboard

- If you have an external midi keyboard, you can plug it into the "Midi In" port of your sound card. The midi code from the external keyboard will control the soundcard. You can select from any one of 127 different "patches" (instruments). One advantage of this is that if you have a good soundcard (one that uses Wavetable lookup rather than FM synthesis to generate sounds) you can make professional sounding music using a very inexpensive external keyboard.

## Mixer

The Mixer Module provides a handy means to control the relative volumes of all the input devices supported by your soundcard. The module is configured to support inputs from a microphone and an auxiliary device (such as a stereo or tape recorder), and from an internal  CD ROM player. Controls are also provided for adjusting the volume of Midi files (using either FM synthesis or wave table lookup technology), and of digital audio (.WAV) files, and for adjusting the final output volume of both right and left stereo channels.

Description of Controls:

Playback Controls

The combo-box lists all the playback devices supported by your computer's soundcard. Click on the down-arrow to display all of these devices, then select the desired device. The slider controls set the playback volume for the Left and Right channel, respectively. The relative values are displayed in the associated boxes next to each slider (range: 0- 256). Checking the Lock check-box locks both sliders so that they move together. If unchecked, you can adjust the playback volume of the left and right channels for the given device independently.

Recording Controls

The combo-box lists all the recording devices supported by your computer's soundcard. Click on the down-arrow to display all of these devices, then select the desired device. Selecting a device automatically sets it as the current recording input line. The slider controls set the recording volume for the Left and Right channel, respectively. The relative values are displayed in the associated boxes next to each slider (range: 0- 256). Checking the Lock check-box locks both sliders so that they move together. If unchecked, you can adjust the recording volume of the left and right channels for the given device independently.

Equalizer Controls

If your computer's soundcard supports equalization, the sliders will allow you to adjust the frequency response in each of a number of different frequency bands. Set the sliders above or below the middle position to boost or attenuate the frequencies in a given band, respectively. Clicking the Reset Equalizers button resets the equalizers to their default position (no boost or attenuation).

About and Help Buttons

These buttons bring up an "About" box and on-line help, respectively.

## Updating the display

- This option redraws the display, using the current shift and magnification values. This is useful if for some reason your display gets messed up.

# The Operations (Effects) Menu

- Accelerando

- Amplify

- Change Pitch

- Compressor

- Echo

- Envelope

- Equalize

- Fade

- Flange

- Filter

- Glissando

- Modulate

- Noise reduction

- Pan

- Reverb

- Reverse

- Ritardando

- Stretch / Tempo

- Spatial Localization

- Tones / Special Effects

- Insert Silence

## Accelerando: Change Tempo

Constant Tempo

With this option you can change the tempo without changing the pitch. The operation will be done either on the highlighted section of the waveform, if there is one, or on the entire waveform, if there is not. You can use the slider control to adjust the change in tempo. For example, setting the slider to +10%, will increase the duration of the selected section by 10%,  thereby slowing the tempo by that amount, whereas setting the slider to -10% will increase the tempo by 10%. Sometimes it might be desirable to relate a percentage change in the duration of a section of the waveform to the actual tempo in beats per second. TsunamiPro has no way of knowing what the actual tempo is unless you tell it. One way to do this is to play a stretch of music and set a time mark for several beats. Then, you can highlight the section between two consecutive beats, and simply read the values of the 'Start' and 'End' at the bottom-right of the main editing screen. The difference between these two values represents the actual time interval for one beat. You can do the same thing for the section of music whose duration you want to stretch or compress. Assume, for example, that one beat is 300 msec for the 'good' section of music, but only 280 msec for the 'bad' section that you want to adjust. To adjust the tempo, you would then choose a value of 300/280 = 1.07, ie, +7%.

Accelerando / Ritardando

With these options you can accelerate or decelerate the tempo smoothly over the duration of the highlighted section. For example, setting the slider to +10% will gradually increase the duration of the selected section by 10%,  thereby slowing the tempo by that amount, whereas setting the slider to -10% will gradually increase the tempo by 10% over the duration of the selection.

See also: Using time markers

## Amplify

To change the amplitude (volume level) of the waveform, first select the desired waveform section (highlighted section), then select the desired amplitude scale factor from the dialog box. If this is a stereo waveform, you can also choose to modify the volume of both the right and left channels, or of either channel separately. The Initial and Final values can be set by moving the mouse cursor over the fader slider knobs, pressing down the left button, and while holding it down, sliding the fader to a new position. (The fader can also be adjusted with the cursor pad arrow keys.)

Normalizing

Selecting the Normalize option will cause TsunamiPro to compute the gain that will cause the greatest sample value to be set to the selected percent of the maximum possible value. All of the other samples will be scaled by this amount, thus assuring that no clipping will occur. For example, choosing a value of 75% will compute the gain factor needed to amplify the waveform to no more than 75% of the maximum sample value. Note that each channel can be normalized to different values. Since TsunamiPro does not display every sample in the waveform (in order to operate as efficiently as possible), it is possible that a single sample value, bigger than all of the others by a sizable amount, will result in all of the other points being scaled to a value significantly less than the percent that you select, and you might not see the single, large sample point. You can use the Magnify function on the View Menu to find and change the values of individual samples.

## Change Pitch

Pitch Options

You have complete control over how much to change the pitch of a selection by, over a range of two octaves (i.e., from 12 flats to 12 sharps, where each sharp or flat represents one semi-tone). The semi-tones are displayed like a piano keyboard, centered on middle C (so,  for example, to increase the pitch by one semi-tone, you would click on the first black key to the right of the black rectangle.) The number of sharps or flats corresponding to a selected key is displayed in the "Value selected" box to the right of the keyboard.

The scrollbar below the piano keyboard lets you select a fraction of a semi-tone. Moving it to the right of center raises the pitch. whereas moving it to the left of center  lowers the pitch. Thus, if you wanted to lower the pitch of the selection by one and one-half semitones, you would select the first key to the left of the black rectangle, and set the scrollbar to -50%

Tempo Options

You can change the pitch of the waveform in two different ways:

1) Constant Tempo

In this mode, the original tempo of the selection is unchanged. You can have the new pitch remain constant by clicking the Constant Pitch radiobutton, or you can have it glide  ("glissando") up or down from the beginning of the selection to the end of the selection by clicking the Glide Up or Glide Down radiobuttons in the Constant Tempo group box, respectively.

2) Change Tempo

In this mode, the tempo as well as the pitch of the selection will be changed. By clicking the Constant Pitch radiobutton,  the waveform is simply resampled at a constant rate, producing a new pitch and a new tempo (e.g., if the pitch is raised, the tempo will be increased, or if it is lowered, the tempo will be decreased). Clicking the Glide up radiobutton causes both the pitch and the tempo to gradually increase from the beginning of the selection to the end of the selection. Clicking the Glide down radiobutton causes both the pitch and the tempo to gradually decrease from the beginning of the selection to the end of the selection.

Scaling Parameters

The pitch-shifting algorithm used by TsunamiPro breaks the selection into a number of small intervals and does an incremental pitch shift on each interval, combining information from the previously processed interval with information from the current interval. The Rate spin control sets the rate of these small intervals (in sampling intervals per second, i.e., it determines the duration of the sampling intervals). The higher the rate value (and the smaller the sampling interval), the more precise pitch change is obtained, but if the rate is set too high, it is possible to overlap information from one interval to the next, producing a series of brief echos. Conversely, if the rate is set too low, some distortion is produced by combining too much information from one interval to the next. In practice, rate values between 20 and 40 are found to produce the best results. Rate values outside this range, however, can produce some interesting audio effects.

The Overlap spin control sets the percent overlap bewteen one sampling interval and the next. Overlap values between 20% and 40% are usually optimal, depending on the frequency content of the waveform. With large Overlap values you can create a "chorus" effect. With small Overlap values you can produce a "choppy" sound effect.

The Use Default Value checkbox sets the rate to a value of 20 Hz, and the overlap to 35%, which are suitable for most waveforms.

Changing Pitch and Preserving the Tempo

Let's say, for example, that your vocalist had a bad night and came in a little flat in a couple of key passages, and you want to goose up the pitch a tad on the flat sections, while preserving the tempo. Simply highlight the section(s), and use the scrollbar to increase the pitch by the required amount, then click the Constant Pitch radiobutton in the Constant Tempo group box, then click OK.

Changing Pitch and Tempo Simultaneously

Changing both the pitch and the tempo simultaneously can create some cool special effects, for example by making a voice passage sound like Andre The Giant (by lowering the pitch several semi-tones), or like Alvin The Chipmunk (by raising the pitch several semi-tones). Simply click on the desired key from the piano keyboard and click on the Constant Pitch radiobutton the the Change Tempo groupbox. You can also make the sound glide up or down by selecting the Glide Up or Glide Down radiobuttons. So, for example, if you selected 2 flats on the piano keyboard and chose the Glide Up button in the Change Pitch groupbox, the sound would start at two flats at the beginning of the selection, and then gradually get higher and faster as it approached the end of the selection.

Compress / Expand Dynamic Range

Description

The Compressor function changes the amplitudes of all samples within a range of levels that you set by drawing on the graph. This makes it possible to increase or decrease the dynamic range of samples, clamp all samples to the same volume, or eliminate samples above or below a threshold value. You can use the Compressor function, for example, to create an even bass or drum line in a waveform that has a group of instruments, or to compensate for the changes in volume of a singer or instrumentalist who moves back and forth near the microphone. The basic idea is to selectively increase or decrease the amplitude of all samples that fall within a specified range. Thus, for example, if the signal made by a flute falls within a quiet range, you can selectively boost all the samples in that range, and enhance the sound of the flute. Conversely, for example, if the signal made by a horn is too loud, you can selectively reduce the amplitude of the loud samples, thus bringing the horn into balance with the other instruments.

Operation

Graphical representation of the waveform data to be compressed or expanded

The graph represents the range of the original waveform values along the X axis (left to right), and the range of the processed waveform values along the Y axis (up and down). If all the processed values equaled all the original values (i.e., if the compressor produced no change in the waveform), then the graph would simply be the white slanted dashed line, in which every point on the X axis is the same as every point on the Y axis.The basic idea is to change the waveform values by drawing on the graph.

Drawing on the graph

A drawing tool pops up when the cursor is in the scratch-pad window, and holding down on the left mouse button starts you drawing at the tip of the drawing tool. To add a point to the graph, simply click on the graph at the location where you would like to add the point. To move a point, simply click on the small square, hold the left mouse button down, and drag the square to a new location.

Scaling the waveform values

Drawing above the white slanted line will increase the amplitude of the waveform values, whereas drawing below it will decrease them. For example, if you wanted to boost the low amplitude values, you could draw a line above the slanted dashed line starting at -100 dB at the bottom left side of the graph, and extending to the point -80 dB (X) and -70 dB (Y), then to the point -60 dB (X), -60 dB (Y) on the slanted dashed line. Then, the low amplitude waveform values would be increased in amplitude by a small amount over this range, making them sound louder.

It is thus very easy to expand or compress the dynamic range of the samples in the selected area. If your plot gets messy, hitting the 'Clear' button will remove all the points you have entered and redraw a straight line representing no change in volume. Selecting 'OK' will apply your hand-tooled custom waveform multiplier to the waveform data, whereas selecting 'Cancel' will  take you back where you came from without scaling the waveform. Clicking 'Save' allows you to save the current values as a named Preset.

Attack

As the compressor does its job, it replaces the original samples with the new values you have entered in the graph. Sudden jumps in the values in the graph can make the scaled sampes make correspondingly big jumps, leading to distortion. To decrease this possible distortion, the compressor limits how fast it will respond to these sudden jumps, by setting an "Attack time". Small values of Attack result in the compressor responding rapidly to changes in the scaled samples, whereas large values of Attack will make the compressor respond more slowly.

Note: the Compressor provides another means of implementing a noise-gate, in which all samples below a noise-threshold are set to zero.

See also:

Noise reduction

Preset Options

## Echo

This option is used to insert echos in the highlighted section of the waveform that you have selected (or in the entire waveform, if no highlighted section has been selected). To set up a desired echo, it is useful to think of what happens when you stand in a canyon and make a loud noise. The noise bounces back and forth on the canyon walls, each time losing energy, so that as it passes your ears, what you hear is a discrete number of repetitions of the noise, starting at an initial volume, and falling off to an inaudible volume. The delay between the echos depends on the spacing of the canyon walls.

TsunamiPro simulates this situation by letting you specify:  1) the Delay, which determines the time between echos in milliseconds, and 2) the Decay, which controls the number of echos.

Short delays (less than 100 ms) produce a "chorus" effect. Intermediate delays (100-300 ms) produce a "slapback" effect, whereas longer delays are perceived as distinct echos.  Selecting "OK" will then generate the echos, "Cancel" will take you back to where you were, and "Save" lets you save the particular echo parameters you have just set as a named "Preset option", so that you can use the same values again without having to reenter them. You can also echo only the left channel or the right channel of a stereo waveform independently by clicking the Left channel or the Right channel radiobuttons, respectively.

See Also Preset Options

## Scale Waveforms With A User-drawn Envelope

This option lets you shape the amplitude of the waveform samples in the highlighted section (or in the entire waveform, if there is none) by hand, simply by drawing in an 'envelope'  on the scratch-pad provided. A drawing tool pops up when the cursor is in the scratch-pad window, and holding down on the left mouse button starts you drawing at the tip of the drawing tool.  The width of the scratch-pad represents the width of the selected section of the waveform. The red horizontal line represents the present amplitude of the waveform section (ie, unity). Drawing above this line will increase the amplitude of the data values (up to a scale value of x 2), whereas drawing below it will decrease them (down to a scale value of x 0). It is thus very easy to create some fancy fade-ins and fade-outs. If your plot gets messy, hitting the 'Clear' button will redraw it. Selecting 'OK' will then apply your hand-tooled custom waveform multiplier to the waveform data, whereas selecting 'Cancel' will  take you back where you came from without scaling the waveform.

You can apply the scaling to both channels, or to either the left or the right channel separately, using the controls provided, for stereo waveforms.

## Four Channel Parametric Equalizer

This option calls up the TsunamiPro four-band parametric equalizer, which can be used to adjust the frequency components in up to four frequency bands simultaneously.  The graph may seem a little esoteric to those not used to working with analog equalizers, but the basic idea is very simple: for each band, you want to set the center frequency to equalize, the width of frequencies about the center frequency, and the amount by which to adjust these frequencies (ie, the gain). The relevant Frequency, Gain, and Width slider controls are provided for each of the four bands. The controls are calibrated in terms of db and kHz. A 3db change in gain represents approximately a 2-fold change in volume.  A 0 db change represents unity gain, ie, no change in volume, so if you set, say, three of the four channels to 0 db, they will not affect the signal at all. Whereas most parametric equalizers display the frequency width in terms of log units, it's much easier to picture what the filtering operation will consist of if you can actually see the frequency width superimposed on the frequency scale in units of kHz, as shown in the plot.

Note that selecting the 'Save' button will allow you to make the particular configuration of frequency control parameters that you have just selected into a 'Preset' option, so that you can perform the same filtering operation on another stretch of waveform without having to set up all of the parameters again.

See also Preset Options

## Graphic Equalizer (10 channels)

Frequency Boost

Sometimes it is desirable to either boost (accentuate) or attenuate selected frequencies in the sound file. For example, most .MP3 compression algorithms automatically boost high frequencies, whether you want to or not. This adds more "color" to the sound. TsunamiPro gives you the same option, using a graphic equalizer, which splits the total range of frequencies into 10 separate bands. Using the slider controls, you can either boost or attenuate the frequencies in the desired range by a factor of plus or minus 3 dB (which corresponds, in turn, to a gain of plus or minus 2). To reset the sliders to 0 dB (ie, no change in frequency gain), simply click on the "Default" check box.

## Fading The Waveform Volume

To change the amplitude of a waveform using the fade option, first select the desired waveform section (ie, select the highlighted section), then select the desired fade scale factors from the dialog box. The fade routine multiplies the volume of the waveform samples in the selected segment by a linear ramp.

For example, to fade IN a waveform from zero-volume (silence) to full volume, set the Initial scale value to 0 and the Final scale value to 100 percent. Conversely, to fade OUT a waveform segment, set the Initial scale value to 100 percent, and the Final scale value to, eg, 0. The duration of the fade will be determined by the length of the highlighted section. If this is a stereo waveform, you can also choose to modify the volume of both the right and left channels, or of either channel separately.

The Initial and Final values can be set by moving the mouse cursor over the fader slider knob, pressing down the left button, and while holding it down, sliding the fader to a new position. (The fader can also be adjusted by simply clicking the mouse at the desired position, or by using the keypad arrow keys.)

Cross-Fading Two Waveforms

The easiest way to crossfade two files (ie, to have the first file gradually fade out while the second file gradually fades in) is to use the "Crossfade" option on the "Combine Selection With Destination" dialog box. You can also use the Fade dialog procedure to accomplish the same thing, albeit by more of a "brute force" method.

There is an alternative but more complex procedure to fade one sound file into another. Let's assume you have sound file A loaded in one instance of TsunamiPro and sound file B loaded in a second instance of TsunamiPro (ie, two copies of TsunamiPro are running concurrently). Now, you want to have sound file A gradually blend into sound file B. Simply fade OUT sound file A, using an Initial value of 100% and a Final value of 0%, and fade IN sound file B over the same length of time, using an Initial value of 0% and a final value of 100%. Next, use the Copy option to copy to the clipboard the section of sound file A you just faded. Then, set the vertical cursor at the point on sound file B where you want to start blending the files, and use the Mix option to mix the two files. The result: sound file B now contains the section of sound file A which gradually fades out while the original music in sound file B gradually fades in to full volume.

You can use a similar procedure to create a sound file consisting of first sound file A, then sound file B, joined together by cross fading. In this case, simply use the Silence option to insert blank space at the start of sound file B equal in length to sound file A minus the amount of 'overlap' where they will be faded together. Then, fade OUT the tail end of sound file A, and fade IN an equivalent length of sound file B at the end of the blank section, then use the Copy and Mix options to cross-fade sound files A and B as before. Now sound file B will start with sound file A, whose tail end will gradually blend in with the beginning of the original sound file B.

See also:

Copy

Mix

Silence

Combine selection with destination

## Flange / Chorus

"Flanging" refers to producing a time-varying phase delay in an audio signal. This was originally accomplished by playing two tape recorders with identical signals, and varying the phase of one of the signals by applying light finger pressure on one of the tape reel flanges. The effects thus produced can result in some totally cool sounds, including a "whooshing" effect (for small phase shifts), a "chorus" effect sounding like multiple voices playing at once (for longer phase shifts), as well as vibrato, echo, and even harmony effects (for still greater phase shifts). To simulate the flanging effect digitally, ideally one would like to be able to independently adjust the amount of phase shift and the rate at which the phase shift changes, as well as the amount of the phase-shifted signal that is mixed with the original signal. TsunamiPro provides these capabilities, and also provides some prerecorded flange settings that produce some interesting sound effects.

To create your own flanging effects, the Set FLANGE Parameters dialog box provides 5 controls.  This might seem a little bewildering at first, but once you understand the concepts involved in creating a flange effect, its all pretty simple.

Delay

The Delay control sets the mean phase shift delay in msec. Delays less than 10 msec produce  "Chorus", "whooshing" and "dive bombing" effects, whereas delay values greater than 10 msec begin to introduce echo effects.

Rate

The Rate control sets the frequency (in Hz) at which the phase is shifted back and forth.  (Low rates, less than 3 Hz, for example, produce a sensation of the sound sources actually moving around in the background, whereas higher rates, above 20 Hz, begin to produce a "warbling" effect).

Depth

The Depth control sets the percent by which the phase delay is shifted about its center value. The higher the Depth value, the more pronounced the flanging.

Feedback

The Feedback control sets the percent of the phase-delayed signal that is fed back to the flanging algorithm (ie, it determines how much "flanging" occurs).

Mix

Finally, the Mix control determines what proportion of the flanged signal and the original signal are actually combined to produce  the output audio signal. A value of 100% specifies that the output signal will be entirely the phase-shifted signal, whereas a value of 0% specifies that the output signal will be entirely the original signal. A value of 50% specifies that the output signal will consist of 50% original signal and 50% phase-shifted signal. Negative values of Mix specify that the phase shifted signal will first be inverted before being combined with the original signal.

## Filter Options

TsunamiPro provides a number of special-purpose digital filters for eliminating noise, modifying the frequency components of a waveform, and creating special effects. The "Freq" slider control is used to set the center frequency for the specified filtering operation, and the "Cutoff" control is used to set the range of frequencies about the center frequency to include in the filter operation. The Filter Response graph depicts the values chosen for a given filter.

The Hash filter eliminates high frequency noise. This filter uses pre-defined values for the center and cutoff frequencies that cannot be changed by the user.

The Low pass filter passes all frequencies below the specified frequency, and attenuates frequencies higher than the specified frequency with a Q proportional to the chosen cutoff frequency (that is, it STOMPS them). It can be used, for example, to emphasize the bass response (by mixing a low-passed signal with the original signal).

The High pass filter does the converse: it passes frequencies above the specified frequency and stomps on those below it. It can be used, for example, to emphasize high frequency response (by mixing a high-passed signal with the original signal).

The Band pass filter passes frequencies in a narrow band (set by the cutoff frequency) above and below the specified frequency, while eliminating those farther away from the center frequency. It can be used, for example, to boost a specific frequency (by mixing a band-passed signal with the original signal). The Sweep option (enabled only when the Band pass button is checked) performs a Filter Sweep on the selected data. Using the Start and Stop sliders to set the initial and final frequencies for the sweep, you can then cause TsunamiPro to perform a series of band-pass filter operations on the highlighted data, starting at the initial frequency at the beginning of the selection, and smoothly gliding to the final frequency at the end of the selection. This can create some very interesting musical effects.

The Notch filter passes all frequencies except those in a narrow range (about 2 Hz) above and below the specified frequency. It can be used to eliminate, for example, 60 cycle hum, or any other constant-frequency component in a waveform.

The Noise gate filter eliminates (ie, sets to zero volume) all portions of a waveform whose peak amplitude falls below a specified percentage of the maximum possible value of an individual waveform data point. This is useful for eliminating background noise during quiet sections of a sound file that contain no musical information.

Note that most of these filtering operations can also be achieved using the four-band parametric equalizer. However, these digital filters are in general faster than the equalizer, and are specifically tailored to produce the desired results. Many of the Preset Options provided with TsunamiPro use these digital filters to produce their effects.

See also:

Four Band Parametric Equalizer

Noise Gate Filter

Preset Options

## Sinusoidal Modulation (Tremulo)

This option modulates the amplitude of the waveform in the highlighted section, if there is one, or in the entire waveform if there is not. This can be used to create some special effects, ranging from the 'wa-wa' like sound of a trumpet mute, to a 'stutter' effect, depending on the rate and the depth of modulation.

The 'Frequency' slider and associated spin arrow buttons set the basic frequency, in Hertz (ie, cycles per second, not the rental-car guys), or you can type in the desired frequency in the edit control below the slider. Frequencies higher than about 5 Hz begin to introduce a quavering sound, and still higher frequencies can lead to aliasing (similar to the wagon-wheel effect in old films, in which the camera shutter sampled the wagon spokes at too low a rate, and they appear to revolve backwards), producing some interesting musical effects.

The 'Depth' controls determine how much modulation (ie, the percent change in amplitude) occurs. A value of 0% produces no change in the signal, whereas a value of 100% makes the signal amplitude go from full value to zero and back every cycle of the sine wave.

You can apply the modulation to both channels, or to only the right or left channel using the indicated controls, for stereo waveforms.

See also: Reducing Noise

## Noise Reduction

TsunamiPro provides a number of features to selective reduce or remove unwanted noise in a waveform.

Hash Filter

The Hash filter eliminates high frequency noise.

60 Cycle Notch Filter

The Notch filter passes all frequencies except those in a narrow range (about 2 Hz) above and below the specified frequency. It can be used to eliminate, for example, 60 cycle hum, or any other constant-frequency component in a waveform.

NoiseGate Filter

The Noise gate filter eliminates (ie, sets to zero volume) all portions of a waveform whose peak amplitude falls below a threshold level. This is useful for eliminating background noise during quiet sections of a sound file. that contain no musical information. The basic idea is that, during 'lulls' (ie rests, or quiet portions of a section of music) you can oftentimes hear objectionable background noise due to amplifier noise, audience noise, stray fire engines, etc) that has no musical content, whereas during 'loud' sections of the actual music, this background noise is effectively masked out by the mysteries of auditory psychophysics. The strategy is therefore to set the background levels to zero, thereby eliminating the audible noise. To accomplish this, TsunamiPro has a built-in lull-detector. All you have to do is to specify, using the slider and spin arrow controls provided, what percentage of the peak height to use as the criterion for background noise. This is of necessity an empirical choice, since the value depends on the actual noise level, the recording amplitude, and other factors. So you may have to make a couple of passes on a test section of music that contains some noisy lulls to get the optimum value. Too low a value, and you miss the noise. Too high a value, and you eliminate some of the actual music. Just the right value, no noise!

Noise Reduction Filter

With this option, you use a statistical sample of the noise  to generate a tailor-made noise filter. Background noise and most types of broad-band noise can be reduced dramatically with little loss of signal quality (typically, noise can be reduced 21dB and signal reduced only 1dB). The noise reduction filter can be used to remove any type of noise (including 60 cycle hum, tape hiss, amplifier noise, and background microphone noise, and it can even remove pops and clicks!) that remains fairly constant in the highlighted section of  your waveform. To reduce the noise from your sound board, simply record several seconds of silence and use that to characterize the amplifier noise. Noise reduction consists of two steps:

1) Characterize Noise ("Sample")

First, the filter must be told what type of noise to remove, which you accomplish by highlighting a short (typically, only a few milliseconds) section of the waveform containing mostly background noise and little if any desired signal. (If you are removing clicks and pops, highlight only a very small segment surrounding a representative click, so as not to include any of the non-click data.) Then click on 'Sample Noise', and the program will obtain a statistical description of the noise in the highlighted section. Then, highlight the section of the waveform that you want to remove noise from, and select the 'Remove Noise' option. Note: you can sample the noise on either the left channel or the right channel for stereo waveforms, by clicking the appropriate radiobuttons.

2) Remove noise

After acquiring a sample of representative noise, and highlighting the section from which to remove noise, set the slider to the desired level of noise reduction. A setting of zero will remove no noise, whereas a setting of 100% will remove the maximum amount of noise (typically, 20dB). If the resulting signal sounds too distorted, use a smaller value. By setting the 'Sample' noise section to a non-noisy section of your sample, the noise reduction filter can also be used to create some interesting special effects. You can remove the noise from either the left channel or the right channel, or both channels, by clicking on the appropriate radiobuttons.

You can repeat the "Remove Noise" step above several times, and compare the sounds of successive iterations. Also, you can use the noise sampled with the "Sample Noise" option on one waveform, to remove the same noise from any other waveform.

Note:

Because of the limited dynamic range of 8-bit audio samples, you can't reduce the noise level more than about -45dB, which is still quite audible. However, if you convert the waveform to 16bit samples, remove the noise, and reconvert to 8bit samples you can increase the amount of noise removed.

See also: Get Rid Of Pops And Clicks

Noise Gate Filter

The Noise Gate Filter is used to remove background noise. The basic idea is that, during 'lulls' (ie rests, or quiet portions of a section of music) you can oftentimes hear objectionable background noise due to amplifier noise, audience noise, stray fire engines, etc) that has no musical content, whereas during 'loud' sections of the actual music, this background noise is effectively masked out by the mysteries of auditory psychophysics. The strategy is therefore to set the background levels to zero, thereby eliminating the audible noise. To accomplish this, TsunamiPro has a built-in lull-detector. All you have to do is to specify, using the slider and spin arrow controls provided, what percentage of the peak height to use as the criterion for background noise. This is of necessity an empirical choice, since the value depends on the actual noise level, the recording amplitude, and other factors. So you may have to make a couple of passes on a test section of music that contains some noisy lulls to get the optimum value. Too low a value, and you miss the noise. Too high a value, and you eliminate some of the actual music. Just the right value, no noise!

Pan Waveform

This option is available for stereo waveforms. The basic idea is to gradually change (either increase or decrease) the volume on one track, while doing the converse operation on the other track, so that the sound appears to move from right to left, or vice versa. Selecting the 'Pan L->R' button results in the sound being panned from left to right, whereas the 'Pan R->L' control pans in the opposite direction. Two controls are available for adjusting the Initial and Final values of the panning operation. Assuming that you are panning from left to right, setting the Initial value to zero and the Final value to 100% results in the left track starting at zero volume (ie, silence) and going to full volume in a linear increase over the duration of the highlighted area, while the right track starts at full volume and goes to zero over the same interval. Intermediate panning effects are produced by selecting intermediate Initial and Final values.

## Reverberation

This option is used to insert reverberations in the highlighted section of the waveform that you have selected (or in the entire waveform, if no highlighted section has been selected).

Reverberations are similar to echoes, in that acoustic energy emanting from a source in an enclosed chamber bounces back and forth off the chamber walls, ceiling and floor (as well as other surfaces in the chamber), and some of these echoes pass through the observers position. In a large chamber such as a cathedral, the spacing between the walls is great enough so that one can distinguish separate echos, whereas in a small chamber, the echoes arrive so close together that they cannot be perceived as distinct echoes, although they still significantly change the quality of the perceived sound. Depending on the chambers surfaces, some of the acoustic energy is either absorbed by the surface, or if the surface resonates to the sound energy, some acoustic energy may actually be added to the reflected sound. Again, depending on the chamber's surfaces, some of the frequency components of the sound impinging on a surface may be selectively absorbed. Typically, in a concert hall with strategically placed sound baffles, the high frequency components are more readily absorbed than are the low frequency components.

TsunamiPro allows the user to simulate many types of reverberations by using the supplied sliders to set the parameters discussed above for an imaginary room (or chamber). You can set:

1) The apparent Room size (which will determine the delays between successive reverberations);

2) The Reflections (which determine how much acoustic energy bounces off the rooms surfaces: the livelier the room, the more energy bounces off the walls, whereas the softer the room, the less energy bounces off the walls);

3) The Frequency Absorption (in Hertz [cycles per second]: this sets the frequency components of the acoustic energy that get selectively absorbed by the room's surfaces;

4) The Dry mix: i.e., the percent of the original sound to be added to the reverberation, and;

5) The Wet mix, i.e., the percent of the computed reverberation to be added to the final output.

You can also selectively reverberate either the Left channel, the Right channel, or Both channels, by clicking on the appropriate radiobuttons.

Selecting 'OK' will  generate the specified reverberation, 'Cancel' will take you back to where you were without generating the reverberation, and 'Save' lets you save the particular reverberation parameters you have just set as a named 'Preset Option', so that you can use the same values again without having to re-specify them.

## Reverse

This options turns the highlighted selection around in time, so that it plays backwards.

## Quasi-3D Sound Placement

Spatial Localization

This option allows you to set the apparent origin of the sound in 3 dimensional space.

The localization algorithm works best on identical waveforms stored in both the right and left stereo channels. The effects are more pronounced using stereo headphones. There are several ways of "placing" the sound:

Far Left or Far Right

This option makes the sound appear to come from either the far right or the far left.

In Between

With this option you can set the apparent origin of the sound to be anywhere along a 180 degree arc in front of the listener with the horizontal slider.

Pan Right or Left, or Back and Forth

These options make the sound move from left to right, right to left, or back and forth, at a rate set by the associated spin controls.

Delay, Invert Left, Invert Right

These options add special effects to the sound.

## Generate Tones and Special Effects

TsunamiPro provides the ability to create spectacular special sound effects, by generating combinations of tones, and by using different tones to modulate the amplitude of waveforms.

Base Frequency

Set the Initial and Final frequencies that you wish to hear. For example, values of 440 Hertz (cps) and 1000 Hertz will produce a tone that starts at the beginning of the highlighted selection at 440 Hertz and changes smoothly to 1000 Hertz at the end of the selection.

Type

Choose the basic flavor of waveform you desire (cosine, sine,square wave, ramp, sawtooth).

Vibrato

The Frequency controls the rate (in Hertz) at which the base frequency is modulated, and the Amplitude determines how much the waveform is modulated by.

Harmonics

Harmonics are overtones that occur at integer multiples (ie,octaves) of the fundamental frequency. They add to the richness of the sound. The Constant option adds  a steady amount of each harmonic to the base frequency. The Warble option makes the amplitudes of the selected harmonics vary between the Initial and Final settings over the duration of the highlighted selection, producing some very interesting sounds. The Swap option exchanges the Initial and Final harmonic amplitudes, and allows  you to smoothly change the sound at the end of the previous selection to a new value.

Phase

The Phase control sets the relative phasing between the left and right channels, for stereo waveforms.

Volume

The Volume controls set the relative amplitudes of the tones generated on the right and left channels.

Generate Tones

This option generates the combination of tones that you have set with the other controls, and replaces the contents of the highlighted selection with these tones.

Modulate Music

This option uses the combination of tones that you have set with the other controls to modulate the amplitude of the waveform in the highlighted selection. It makes very strange and interesting things happen to a musical selection.

Save

This option lets you save the current settings as a Preset.

Preset options

## Insert Silence

This option is used to set waveform samples to zero (ie, to create silence). There are two modes of operation.

In the 'Equal to length of selection' mode, a new set of data values of zero amplitude, equal in length to the duration of the highlighted section, if there is one, or to the entire waveform if there is not, will be inserted into the waveform at the current position of the vertical cursor. This is a useful method for creating a silent gap between two selections.

In the "Select minutes and/or seconds" mode, a length of silence that you set in the "Minutes" and "Seconds" controls will be inserted into the waveform, lengthening it by that amount.

# The DirectX Menu

TsunamiPro supports (hosts) DirectX Plugins (transforms), a variety of which are available from third-party software vendors, as well as from Black Diamond Sound Systems. A DirectX Plugin performs a specialized transform operation on the highlighted section of a wave file. To Register a new plugin with TsunamiPro (which supports self-registration), simply drag the icon of the plugin onto an active screen of TsunamiPro and drop it there by letting go of the left mouse button. A dialog box will then show the progress of the registration process. Use the 'Add' button to select as many plugins as you like: the plugins will be chained together in the sequence that you have selected them. When you are finished selecting the desired plugins, click the "Next" button to display the property page for the first selected plugin.

DirectX Plugin Example

Shown above is an example DirectX Plugin ("Gargle", available with the Microsoft Direct Show Software Developer's Kit). The 'Back' button takes you back to the Select-plugins dialog box. The 'OK' button actually performs the plugin's transform on the highlighted section of the waveform. The 'Cancel' button return to TsunamiPro without modifying the waveform. The 'Preview' button performs the plugin's transform, but does not modify the waveform. The 'Add' button allows you to store the current values you have set for the transform as a preset. The 'Delete' button deletes a previously stored preset selected in the 'Presets' combo-box edit control.

# Recording a Waveform

TsunamiPro provides a separate dialog box which provides complete flexibility in recording a sound file. When you click on the 'Rec" control on the main editing window, the record dialog box is activated.  When you first activate this dialog box, TsunamiPro is ready to record to  a new sound file., which it will save as 'Samplex.wav'. If you wish to record to a preexisting sound file., press the Wavefile button and select the desired file.  All recording will then be done to a copy of that file. Although it takes a little longer to create the copy, by recording to a copy you are protected against accidently messing up a valuable original file.

Recording a New Sound file

Before starting to record a new waveform, you have the option of setting the format in which it will be recorded, by toggling the appropriate Resolution (8 bits or 16 bits), Channels (mono or stereo), and Sample Rate (11.025, 22.050, 44.100 kHz) buttons. Each time you select a new recording format in this fashion, the Disk Space window will be updated, showing the total number of minutes and seconds available on the disk for recording at the given format.

Rerecording To An Existing Sound file

As previously mentioned, you can rerecord to a sound file. stored on your hard disk. Select the desired file by clicking on the Wavefile control. You can also rerecord to a newly-recorded sound file. When recording to an existing waveform, the format buttons will be set to reflect the format of that waveform, but they cannot be changed. (The Windows sound file. drivers won't let you change formats in mid-stream.) You can, however, change the format of the entire sound file. after you have finished recording it, by using the 'Save as' option on the Files menu.

Rerecording will begin at the location selected by the scroll bar at the bottom of the screen, or, you can play the file up to the point that you want to begin rerecording, or you can use the Review and Cue buttons to position the point at which to insert a newly recorded section. Note that when rerecording to an existing file, you have two options for how the newly recorded data will be incorporated into the file, specified by the Punch-in Mode buttons. In the Overwrite mode, the newly recorded sound will replace the original sound that was at the same position in the waveform. In Insert mode, the newly recorded sound will be inserted into the original waveform at the point that you select. When you stop recording in Insert mode, the rest of the original waveform will be copied to the end of the recording.

Adjusting the Recording Level

You can preview a recording and set the recording input level, by pressing the Test button. In Test mode, you will hear the input signal, and see the level displayed on the LED meter, but the samples will not be saved. In general, the best signal-to-noise ratio is obtained by setting the input recording level as high as possible. Because of the wide dynamic range of audible sound, however, there is a high probability that some of your samples will exceed the dynamic range of the analog-to-digital converter on your soundcard if the recording level is set too high, causing distortion which you will find to be truly objectionable. For this reason, the signal level should be adjusted (using the Record Level slider) for the mid-range of the LED meter (ie, at the junction between green and yellow) for best results. If the signal level is too high, the Clip meter will flash red. Remember to turn off the Test mode function before actually recording. To turn off the Test mode, click on the Stop control. The recording level for any input source supported by your soundcard can be set with the Mixer, as explained in the detailed help for the Mixer.

Selecting the Input Source(s)

TsunamiPro can record from a number of input sources, including a microphone (Mic), an CD-ROM player, a Midi file being played on the soundcard, or an external device connected to the line-in or aux-in jack on the soundcard. To "connect" these inputs to the digital waveform recording circuitry, simply click on the desired device in the Record Source Group Box. You can also use the built-in  Mixer, or the mixer application that came with your soundcard. Clicking on the 'Show CD' control brings up the CD Player, which provides a convenient means of playing any track on a CD. Clicking on the 'Show Midi' control brings up the Midi Player. For details on how to use the Midi player, click the Help button on the Midi Player. The 'Trigger' control on the Midi Player lets you synchronize recording and playback with a midi file. This control is disabled until you have read in a midi file on the Midi Player. Synchronization is turned on or off by toggling the control.

Acquiring Samples

Once you have specified the recording source, set the format, and adjusted the recording level, you're ready to record. Simply press the Record button, which will turn red, indicating that you are now recording. The amount of time elapsed from the start of the newly recorded file will appear in the Time window, and the total length of the recorded file will be continuously updated in the Length window. To stop recording, simply press the Stop button. The blue position bar and the scroll bar at the bottom of the screen now appear at the position in the waveform that you pressed stop. If you messed up or are not satisfied with a given recording, you can press the New button, which will begin a new recording.

Metronome

TsunamiPro provides a built in metronome that can be used as an adjunct in live recording. You can set the beats per minute (before actually recording) with the spin controls. The metronome will then click and flash red at the desired rate during Test or Record mode. The rate cannot be changed during actual recording. Using the metronome will alter the speed at which the record level and dB meters are updated.

Playing Back The Recording

To play back a recorded file, use the scroll bar or the Review and Cue buttons to select the position at which to begin playback. You can also use the arrow buttons next to the scroll bar to go to the beginning or end of the file. To start playback, simply press the Play button, which will then turn green. You can interrupt or resume playback with the Pause button. You can scan forwards or backwards in the currently playing selection in increments of one second by using the 'Cue' and 'Review' controls.  Double-clicking on these controls (ie, pressing the left mouse button twice in rapid succession) and holding down the left button causes continuous skipping.

Auto-rewind

You can toggle on or off the auto-rewind. When auto-rewind is enabled, the file position marker will automatically return to the beginning of the file after recording. When auto-rewind is disabled, the file position marker will remain at the current position when you stop recording or playing. New samples can be recorded to the file at that point.

Auto-stop

This option let's you set the recorder up to record for a specified length of time and then automatically stop recording. In conjunction with the sampler feature, this makes it very convenient to work away from your computer. You can set up the recorder to start recording when it gets a specified input signal level, and then to automatically stop recording when desired, without ever having to touch any of the controls.

Synchronizing Recording or Playback

The 'Sync' option allows you to synchronize recording or playback with an external timing source, as explained under the 'Synchronize Recording or Playback' help topic. With this option, TsunamiPro will not begin recording or playing a waveform until the amount of time you have specified has elapsed. If you are  recording from or playing back to an external tape drive that has a timing track recorded on it, and you have the hardware to convert this signal into a standard SMPTE or MIDI timing signal, the Sync option can then be used to transfer data to or from the tape at specific locations. This allows you, for example, to record a section of music from the tape drive, edit the recorded music, and put the edited waveform back onto the tape at the same location. You can also generate a free-running SMPTE time code that can be used, for example, to lay down a SMPTE track on an external tape recorder, or to synchronize other instruments.

Simulating a Digital Sampler

The 'Sample' option simulates a digital sampler, as explained under the 'Digital Sampler' topic.

Saving A New Recording

To exit from the record dialog box back to the editor, you have two options. If you press the Exit button, you will return to the editor WITHOUT SAVING the newly recorded file, if any. To return to the editor with the newly recorded file, press the OK button. TsunamiPro will then replace whatever file you might have been previously editing with the newly recorded file. You can use the Preferences dialog box to set the total amount of visible display time. If you have just recorded a 100MByte file, and you have set the Preferences to show the entire waveform it will obviously take a long time to display the entire file, so you might want to set the Preferences to display only the first 10 seconds or so of the file, which will take only a few seconds.

Show CD Player

This brings up the TsunamiPro CD Player module at the bottom of the screen. You can record directly from CD's using this option.

Show Midi Player

This brings up the TsunamiPro Midi Player. You can synchronize recording and playback with midi files. The Midi Player allows you to select specific measures and SMPTE times within a midi file, and also provides a lead-in delay, so that you can easily record an accompaniment to a midi file.

Mixer

This pops up the TsunamiPro Mixer, which makes it convenient to change the volume settings for wave file playback, midi files, the CD player, and the gain of the auxiliary input on your soundcard.

## Waveform Format

Before starting to record a new waveform, you have the option of setting the format in which it will be recorded, by toggling the appropriate Resolution (8 bits or 16 bits), Channels (mono or stereo), and Sample Rate (11.025, 22.050, 44.100 kHz) buttons. Each time you select a new recording format in this fashion, the Disk Space window on the Record dialog box will be updated, showing the total number of minutes and seconds available on the disk for recording at the given format.

### Change / Set Waveform Format

This option brings up a dialog box that lets you examine and /or change the format of a waveform.

Use this option when you want to create a new waveform file, to specify whether the new waveform should be 8 or 16 bits resolution, mono or stereo, and what sample rate it should have. Note that the changes that you make do NOT affect the waveform that is currently being edited. In order to change the format of the waveform that is currently being edited, use the 'Save file (new format)' option under the 'File' menu.

## Digital Sampler

This option simulates a digital sampler. When you invoke the Sampler (by clicking on 'Sample' on the Record Dialog Box) a small dialog box will pop up and ask you to enter two parameters that will control when the sampler starts and when it stops.

The 'Threshold' value sets the threshold volume of the input signal that will trigger the digital sampler to start sampling. The range is from 0 to 100 % of the maximum dynamic range of the A/D converter of the soundcard.

The 'Duration' value sets the length of time (in seconds) that the Sampler will sample the input signal after it is triggered. The range is from 0 to 1000 seconds. You can override this value and set the Recorder to record continuously by checking the "Continuous" check box.

Operation

After you set the threshold and time parameters and return to the Record Dialog Box, the recorder is 'armed' and ready to start recording. When the voltage on any of the selected input sources exceeds the threshold setting, the Recorder will begin recording. Note that the format of the sampled waveform is set by the controls on the Record Dialog Box. The sample control turns red during the recording period that you have set up. Sampling will automatically stop when the number of seconds that you specified have elapsed, or you can terminate recording at any time by simply pressing the 'Stop' control.

## Synchronize Recording or Playback

Synchronizing Recording or Playback With An External Midi or SMPTE Time Signal

The 'Sync' option allows you to synchronize recording or playback with an external timing source.  With this option, TsunamiPro will not begin recording or playing a waveform until the amount of time you have specified has elapsed. If you are  recording from or playing back to an external tape drive that has a timing track recorded on it, and you have the hardware to convert this signal into a standard MIDI timing signal, the Sync option can then be used to transfer data to or from the tape at specific locations. This allows you, for example, to record a section of music from the tape drive, edit the recorded music, and put the edited waveform back onto the tape at the same location.

To be able to use the Sync option, you will need to connect the timing signal from your tape drive to the appropriate hardware  to convert it to a standard MIDI timing format.  Using a standard MIDI cable, connect the MIDI OUT from the converter to the MIDI IN cable on the midi connector on the soundcard that you have selected as the I/O input device. You will then be set up to detect MIDI timing signals from the tape drive.

Synchronize to MIDI Clock or to SMPTE time code

Most modern recording studios use a timing technique called "SMPTE" to synchronize musical and video events. SMPTE (which stands for "Society of Motion Picture and Television Engineers") is a timing standard that was originally developed to synchronize the audio track on motion pictures, in which time (elapsed since some reference time, such as the beginning of a video tape or film) is represented in the format:

hours:minutes:seconds:frames

Motion pictures commonly consist of 24 individual "frames" (ie, separate pictures) per second, which, being faster than the human visual flicker fusion frequency, is perceived not as a jerky series of discrete images, but rather as a smoothly varying continuous flow of visual images. This is the SMPTE 24 format. In Europe, the AC electrical current is 50 cycles per second, and the standard frame rate for the PAL and SECAM television systems is 25 frames per second, or SMPTE 25. In the USA, the AC electrical current standard is 60 cycles per second, giving rise to the SMPTE 30 (ie, 30 frames per second) standard. To make things weirder, the NTSC (National Television Standards Committee) set their frame rate 0.1% slower in order to avoid an annoying interference in the color sub-carrier portion of the standard television signal. To accommodate this 0.1% difference, the SMPTE 30D time code was introduced, in which at the beginning of every 20th minute, 2 of the 30 frames are "dropped" (ie, frames 0 and 1 are skipped). TsunamiPro lets you sync to any one these four standards, simply by clicking on the appropriate radio-button.

You can synchronize either to external MIDI clock signals, or to an external SMPTE time track. When synchronizing to an external MIDI clock source, simply set the time delay (in seconds) that you want to wait before beginning to record or to playback a waveform. When synchronizing to an external SMPTE signal, set the frame that you want to start recording or playing back from, either by typing in the values or by using the spin controls. The Sub frame can be used to provide further timing accuracy. It will count for up to an additional 1/20th of a frame before starting playback or recording. Note: the program expects to get its sync signals from the MIDI IN cable connected to your sound card. Before using the Sync option, you should make sure that your Midi Input device is "connected" to the Midi In port of your sound card, using the Select MIDI I/O Devices option from the Setup menu.

Triggering Recording or Playback

The options included in the Trigger group box allow you to trigger recording or playback from an external SMPTE source (such as an external tape recorder, using an MQX32M board), from a sequencer program using midi time code (MTC), from an external midi source plugged into the midi-in port, or from the TsunamiPro Midi Keyboard player. The Event, Channel, and Note controls let you specify what type of midi event (note on, note off, or note playing) will trigger playback, as well as what channel and what note to use. Use the Midi Devices dialog box in the Preferences menu option to select the appropriate midi output device that will provide the midi trigger, as well as the appropriate midi input device that will receive the incoming midi signal, and make sure that the appropriate connection is made from the selected midi out port to your computers midi in port using a midi cable.

Examples

As an example of using the 'Sync to Midi Clock' option, let's assume that you have a section of music recorded on your tape recorder that you want to edit and to rerecord back to the same position. Assume that it starts at time 0:02:20:00. You could stop the tape at 0:02:10:00 (ie, 10 seconds before the section of interest), and set a 10 second delay with the Sync dialog box in Recording mode. Then, start your tape recorder. When the tape has gotten up to speed and reached the section of interest, TsunamiPro will begin to record. Simply press 'Stop' on the Record dialog box when you have acquired the section of interest, which you can then edit. Save the edited waveform as a named file, and return to the Record dialog box and select the 'Filename' option to read in the file. Set your tape recorder back to 0:20:10:00, and use the 'Sync' dialog box to set a delay of 10 seconds in "Playback' mode. Then start your tape recorder and when it comes to the selected section, TsunamiPro will begin playing back the edited waveform, and you can rerecord it back on to the tape.

As an example of using the 'Sync to SMPTE' option to record:

1) Set the midi input device using the 'Set MIDI I/O Devices' from the Setup menu (eg, "TAHITI EXT IN", for a Turtle Beach Monterey card, or "SB16 Ext IN" for a Creative Labs SoundBlaster 16 card).

2) Plug the cable from your SMPTE time-code source into the MIDI IN receptacle on your soundboard.

3) On the Record dialog box, set up the desired recording format and source.

4) On the Sync dialog box, select the Trigger on SMPTE option.

5) Enter the desired frame you want to start  recording at.

6) Select the Synchronize for Recording option.

7) Enter OK.

The large Time Display window will appear and will display the incoming SMPTE time-code if everything is set up correctly, and when the SMPTE value reaches the value you have set, recording should begin. Press Stop to terminate recording.

See also:

Select I/O Devices

Select MIDI I/O Devices

Recording a Waveform

Synchronizing Recording or Playback

## Wave Player: Four-track Recording and Editing

This option allows you to record a stereo file while simultaneously playing a second stereo file in synchronization, using two soundcards (or a single soundcard, if it supports full duplex operation). This is very useful for creating a multi-track recording.

Description Of Controls

"File": Select Program To Play

This control brings up the Playlist Dialog box, with which you can select either single wave files to play, or a group of wave files to play. You can also save your selection of files as a Playlist, which can then be played over again at a later time.

Adjust Playback Volume

The fader control can be used to adjust the playback volume by simply dragging on the thumb control with the left mouse button held down. If the control has the input focus, you can also use the cursor keys to adjust the volume. Changing the position of the volume control will also update the position of the Wave volume control on the Mixer Module, and vice versa. Thus, the Mixer module doesn't have to be displayed in order to change the playback volume.

Time Display

The time display window indicates the position within the wave file currently being played. The units are minutes:seconds, and the display is updated once per second. Below the display window, the name and length of the wave file currently being played (or currently loaded, ready for play) is shown. The characteristics of the file, including the number of channels (Stereo or Mono), the resolution (8 or 16 bits) and the sample rate, are also displayed.

Select Settings For Playing Wave Files

Use the "Preferences", "Wave Files" from the Files Menu to select which sound boards to use as input and as output in systems that have more than one sound board installed. Simply select the desired soundcard name from the list of soundcard input and output devices displayed in the relevant combo boxes.

Dynamic Display of Playback Volume

This control simulates a standard LED meter control. The relative volume of the left channel is shown at the top, and of the right channel at the bottom, for stereo waveforms. For monophonic waveforms, both the top and the bottom indicators show the same values.

Playback Controls

The operation of the playback controls is self-evident from their labels and standardized bitmaps. To resume playback after clicking on the 'Pause' button, click on the Pause button again, which will be yellow while the playback is paused. The 'Play' control will be green during active play. You can scan forwards or backwards in the currently playing selection in increments of one second by using the 'Forward' and 'Reverse' controls. Double-clicking on these controls (ie, pressing the left mouse button twice in rapid succession) and holding down the left button causes continuous skipping. You can skip to the previous or to the next selection by using the 'Previous' or 'Next' controls.

### Synchronizing Playback With the Recorder: Four Track Operation

This option allows you to use one soundcard to play a stereo file while simultaneously recording a stereo file using a second soundcard (or with the same soundcard, if it supports full duplex operation). This is a very useful feature for building a multi-track recording because you can record all of the tracks in the proper synchronization which would not be possible if only the two tracks of a single soundcard were available.

Operation

The blue bar shows the relative length of the selected wave file. Use the mouse to highlight a section of the wave file to play (hold down the left button and drag across the desired region). The relative start and stop times of the playback are shown in SMPTE-24 format in the corresponding controls. You can set a "lead-in delay" with the spin controls, which will start the wave file playing before actual recording begins (so you can get up to speed if you are doing a live accompaniment).

When you have activated the Triggered control, when you press Record, the wave file will play for the specified lead-in time, then recording will automatically start and continue for the length of time you have selected on the blue bar. Both recording and playback of the wave file will then stop automatically.   You can adjust the synchronization with the Adjust Sync scroll bar. Each click of the scroll bar arrows will be recorded, and will go into effect the next time you press Record on the Recorder.

# Playing a Waveform

Auditioning a Sound file

Clicking on the Play button on the Open File dialog box allows you to listen to a selected sound file without having to first load it into the display.

Playing  a Sound file in a display window

After you have recorded a new waveform or loaded a preexisting waveform from disk, it is ready to be played. You can either play the entire waveform, or a  highlighted section. Simply press the Play button on the transport toolbar or on the playbar at the bottom of the window. You can also simply right-click anywhere on the waveform display, and playback will start at that point in the song file. You can also start playback by pressing 'p' on the keyboard, and stop playback by pressing 'o'. The Pause button can be used to interrupt or resume playback . The Loop playback buttons cause the selected section to be played continuously.

## Play Clipboard Contents

This option lets you listen to what is currently in the clipboard. Simply click on the “Play clipboard” option on the Edit menu (or press "Ctrl y").

# Editing a Waveform

The editing functions in TsunamiPro are closely analogous to those of most text-editing programs, such as Microsoft Word. As in such programs, you start by highlighting a section of the waveform to be edited, by holding down the left mouse button at the start of the section to be edited, and then dragging it to the end of the section to be edited, then releasing the button. (You can change the highlighted selection by repeating the above procedure with the Shift key held down, or by clicking on the Review and Cue buttons, or on the Highlight buttons on the Cursor toolbar.)

## General Editing Operations

Once a selection is highlighted, the Edit menu will indicate that you can then Cut, Copy, Clear, Crop or Trim the waveform. These functions are described in detail in the indicated links. You can also use the Select all option on the Edit menu to highlight the entire waveform.

When you click on Copy (either on the Edit toolbar, or on the Edit menu), the highlighted section is copied to the clipboard, and can then be Pasted, either to another spot in the same waveform, or to a different waveform. This is indicated by the fact that the Paste option on the Edit menu is now active. If you activate another waveform (by clicking on it, or by selecting it from the Window menu, this waveform becomes the "destination" waveform, whereas the original waveform is known as the "source" waveform. If the destination waveform in turn has a highlighted selection, the Edit menu will show that you can Paste the contents of the clipboard to it, which will replace the highlighted section of the destination waveform with the contents of the clipboard. (You can use the Undo and Redo buttons to undo or redo this or any other editing operation.) If the destination waveform has no highlighted selection the Edit menu will indicate that you can "Paste insert", "Replace", or "Mix" the contents of the clipboard onto the destination waveform. All three of these operations take place starting at the position of the caret on the destination waveform (which you set simply by clicking on it at the desired location). "Paste insert" inserts the contents of the clipboard (ie, it copies the clipboard to the destination waveform starting at the caret, then copies the rest of the waveform, resulting in a waveform that is longer than the original one.) "Replace" is just like Paste with a highlight:: the contents of the destination highlight are replaced by the contents of the clipboard, but does not change the length of the file. "Mix" mixes the contents of the clipboard with the contents of the destination waveform in equal proportion, for a length equal to the length of the contents of the clipboard.

You can also accomplish all of the above operations using drag-and-drop from one waveform to another. For example, assume that you have highlighted a selection on Waveform A. If you then click with the left mouse button anywhere within the highlighted selection, and, holding the left button down, drag it over Waveform B and then release the mouse button, the Combine Selection With Destination dialog box pops up, which gives you the opportunity to Paste, Replace, Mix, or Crossfade, as explained in detail in that Help topic. Note that if the formats of the source and destination waveforms differ, TsunamiPro will do an automatic format conversion. If you release the left mouse button over a blank section of the TsunamiPro screen, you can create a new waveform, consisting of the contents of the highlighted selection. It will have the same format (eg, 8 bits, stereo, 22.5 kHz), as the original waveform.

## Crossfade Two Waveforms

If the destination file has a highlighted selection, the source selection will be added to the destination waveform beginning at the start of the destination selection, and the destination selection will be faded out while the source selection is faded in over this interval. The initial amplitudes of the source and destination data is set with the corresponding fader controls. The fading can either be linear, or exponential (which compensates for the actual sound energy in the two waveforms).

Example:

As a practical example, assume you have two files, named "A" and "B", and you want a new file that starts with file A, then gradually crossfades into file B. First click on file A and highlight the end of the waveform for a length of time equal to the length of time you want the crossfade to take, say 10 seconds. Then click on file B and select "Select All" from the Edit menu. Place the cursor somewhere in the highlighted region of file B and drag it to a position over file A, and then release the left mouse button. Select "Crossfade" from the Combine Selection With Destination dialog box. TsunamiPro will then extend file A such that its original tail end will fade into file B.

# Using Time Markers

As the name suggests, Timemarks are simply time-reference points that you can insert into the sound file. display. To do this, simply click on the Mark button or press 'm' while the waveform is playing, or when the file is not playing and the vertical blinking cursor is present by itself (eg, when there is no highlighted segment selected), and a small triangle will appear at the appropriate position on the top of the display window. You can insert up to 50 time marks in a sound file. Time marks are very useful for a number of editing operations, for example, delimiting measures or sections to copy or delete, or transposing measures.

Accessing Timemarks and Defining Intervals

If you position the mouse cursor over a time mark the data associated with the timemark is displayed on the message bar at the bottom of the main window. Clicking on the right button brings up the timemark properties dialog box.

Timemark Properties Dialog Box

This dialog box lets you define the name of a timemark, create a matching pair for a given timemark (thereby creating  a highlighted interval), and select among several options to trigger playback of the interval defined by a pair of timemarks. To change the name of a timemark, simply enter the desired name in the right or left Marker  combo box. To change the time of the timemark, use the Start time  and Stop time horizontal scroll bars. To define a new interval (ie, a pair of timemarks enclosing a selection), either select an existing marker from the combo box below the "use existing marker" button, or click the "Make new marker" button to create a new marker / interval. Click on the Save interval to store this new interval.

Triggering Playback

The options included in the Trigger group box allow you to trigger playback of a selection enclosed by a pair of timemarks (ie, a "cue"), or trigger playback starting at an unpaired timemark. You can trigger playback from an external SMPTE source (such as an external tape recorder, using a MQX32M board), from a sequencer program using midi time code (MTC), from an external midi source plugged into the midi-in port, or from the TsunamiPro Midi Keyboard player. The Event, Channel, and Note controls let you specify what type of midi event (note on, note off, or note playing) will trigger playback, as well as what channel and what note to use. Use the Midi Devices dialog box in the Preferences menu option to select the appropriate midi output device that will provide the midi trigger, as well as the appropriate midi input device that will receive the incoming midi signal, and make sure that the appropriate connection is made from the selected midi out port to your computers midi in port using a midi cable.

Highlight

The 'Highlight' button brings the region between two time markers into view and highlights the region between them.

Moving Time Marks

To move a time mark, simply place the mouse cursor inside the time mark rectangle, press down on the left button, and move it to the desired location on the display. You can accomplish the same thing by using the Start time and Stop time horizontal scroll bars on the Timemark dialog box.

Clearing Timemarks

To clear (ie, remove) all the time marks from a waveform display, simply click on the Clear button.

Changing Time Mark Colors

You can use the 'Customize Control Colors' option to change the color of the time marker and of the time marker name.

Also see:

Cue list

Multitrack list

# Adjusting the Display: Multiple Editing Windows

TsunamiPro provides a number of controls with which you can adjust the portion of a wave file that is visible in the display window. The portion of a waveform that is visible on the display window is shown (in seconds from the beginning of the file)  by the time-mark numbers above the upper display window. The number of files (editing windows) that you can open at any time is limited only by the available amount of memory.

1.  You can set the length of the display that is initially visible when you read in a stored file or record a new file by using the 'Zoomed startup display' control in the Options tab of the 'Preferences' dialog box, selected from the Files menu. By choosing this option, which sets the zoom factor to the maximum possible value,  you can dramatically decrease the time that it takes to initialize or to redraw the display.

2. You can shift the display backwards or forwards in time by using the X Shift horizontal scroll bar at the bottom of the display window.

3. You can adjust the horizontal magnification of the display by using the Zoom and Unzoom buttons located at the bottom of the display window. The magnification factor is shown at the right of the these two buttons.

4. By pressing down on the left mouse  button while it is inside of the white horizontal time-bar above the display window, and dragging it to a new time before releasing the left button, you can fill the display window with any desired portion of the waveform. This provides a very convenient means to select and view different portions of the waveform.

5. After setting a number of time-markers, you can use the Time Marker controls to rapidly jump to specific time markers, or to highlight the region between a selected pair of time markers. To activate a specific time mark, place the mouse cursor inside the timemark and click the right button.

6. You can use the Review and Cue buttons on the Cursor-control toolbar to extend the duration of the highlighted section of a waveform (in increments of 50 msec). Similarly, you can use the four highlight buttons on the Cursor-control toolbar ( (L highlite <-, L Highlite ->, R Highlite <-, R Highlite ->) to adjust the highlighted section in increments of 10 msec.

7. You can use the Move to beginning and Move to end buttons on the Cursor-control toolbar to move to the beginning and end of the waveform, respectively.

Using Time Markers

Updating the display

# Getting Help

TsunamiPro has an extensive built-in help system. To activate the system when you are working on the main editing screen, simply  press the F1 key. this will activate the main Help System.

To get help when you are dealing with a dialog box rather than the main screen, simply click on the Help button on the dialog box.

# Accelerator Keys

Accelerator keys are specially designated keys that when pressed automatically select a menu item and execute the associated function without the user having to select the item from the menu. Because this says an appreciable amount of mouse clicking  as well as time, accelerator keys are very handy. The key designations appear on the menu items to the right of each function.

In addition to menu-associated accelerator keys, TsunamiPro also has some accelerator keys that are associated with certain bitmap-control buttons on different toolbars. The toolbars do not have to be visible for these keys to work. These keys are:

"p"       Starts playback

"o"       Stops playback

"m"      Enters a timemark at the current cursor location

"z"       Zoom (x 2)

"u"       Unzoom (x 2)

# Preset Options

TsunamiPro provides a number of 'Preset' options for filtering waveforms, for playing Playlists, and for a number of other operations. These operations are accessed through a common 'Preset Options' dialog box, the controls for which are pretty straightforward. The list of relevant preset options appears in the combo-box at the bottom of the dialog box. Simply click on the desired option with the left mouse button to select it, and it should appear in red in the Preset control. Selecting 'OK' will then perform the operation, 'Cancel' will take you back to where you were without performing it, 'Delete' will delete the option, whereas 'Create' will take you to the appropriate routine where you can then create and save a new preset option, which will then be available the next time you want to perform a similar operation. This saves you the bother of repeatedly having to specify all of the parameters for a given operation every time you want to perform it. 'View' will display the relevant dialog box that contains and sets the actual values used to control the selected function.

(Note: the goofy-looking little guy in the purple pants, holding a baton and scratching his head in bewilderment, is the famous Professor Ludwig B. Pflatt, Director of NASA’s Extraterrestrial Music Program.)

Save Preset Values

To save a particular set of values as a 'Preset', simply type the name that you would like to give the preset option into the edit control under 'Save-file name:'.  A list of existing preset names associated with the same operation is shown in the list-box below this, for reference. Activating the 'Save' button will then save the named preset, which can  be called up at a later time to perform the same operation, without having to re-specify any of the control parameters.

# On-Line Tutorial

TsunamiPro contains a built-in On-line Tutorial which demonstrates step-by-step exactly which buttons and controls to activate in order to accomplish a number of common tasks, such as recording a file.

# Windows Clipboard

# Windows ClipboardWindows Clipboard

The Windows clipboard is a block of memory that is used to store data that will be transferred either to or from TsunamiPro or another application. When you click on Copy, Cut, Paste, or Mix, the waveform samples that are within the highlighted section get transferred to or from the clipboard. Windows retains 'ownership' of the clipboard memory. Thus, for example, if you have transferred some data to the clipboard with one instance of TsunamiPro, when you run a second instance of TsunamiPro, these data are  available for being copied, pasted, or  mixed.

# Highlighted Section

All editing and playback is performed either on the user-selected highlighted section, or, if none is present, on the entire waveform. To highlight (ie, select) a desired section for editing or playback, adjust the horizontal shift and magnification parameters so that the desired section is visible in the display area. Then, move the mouse cursor to the start of the desired section and press down the left button. While holding the mouse button down, move to the end of the desired section, then release the left button. The desired section will then appear highlighted. The beginning and ending times for the highlighted section will be displayed in the respective Selection Start and Selection End boxes on the display, in units of either seconds or SMPTE values, depending on which mode has been selected using the Preferences option from the Files menu. The highlighted area can be adjusted in increments of 100 msec by clicking on the appropriate 'spin' button arrows. The section will remain highlighted if the horizontal shift and magnification factors are subsequently changed. To remove a highlighted section (ie, to deselect it), simply move the mouse cursor to any point on the display and click the left button. You can highlight the entire waveform by choosing the Select all option from the Edit menu. You can also change the duration of a previously highlighted selection by holding down the Shift key while dragging the mouse with the left button depressed at the beginning of ending of the selection.

# In Case Of Trouble

If the TsunamiPro Setup program completed the installation procedure without giving any diagnostic error messages, it is likely that whatever problems you might be having are caused either by hardware difficulties, or by difficulties with various Windows MultiMedia drivers. The first thing to check is whether all of the necessary cables are plugged into your sound card  properly. Assuming everything is plugged in where it ought to be, the next thing to check is whether your soundcard is configured properly. The majority of soundcards use a method called "Direct Memory Access" (or DMA) to communicate with the computer, and require that the soundcard be configured at installation time to use one of the DMA channels stipulated by the manufacturer. The soundcard also communicates with the computer via program "interrupts", and the soundcard manufacturer will also list the range of allowable "IRQ" interrupts to use. A problem frequently arises if either the DMA channel or the IRQ interrupt is being used by another hardware device, such as a SCSI controller, a tape-drive, a CD drive, a mouse, another soundcard, or a device plugged into one of your serial or parallel ports.

The symptoms produced by either a DMA or an IRQ conflict can vary, but almost always will affect the ability of your soundcard to read or write data at high speeds. If, for example, you are having problems recording or playing back sound at 16 bits resolution and 44 kHz sample rate, the culprit is likely to be a DMA or IRQ conflict, or both. The first thing to do is to look through whatever notes and documentation you have concerning the DMA channels and IRQ vectors used by all of the devices already plugged into your computer. Then, choose a "free" DMA channel and IRQ vector that is also on the list of allowable settings provided by the manufacturer of your soundcard, and change the soundcard settings to these new values. You can also use a number of commercial hardware- reporting programs (such as WinSleuth Gold, from Dariana Software), to aid you in sorting out all of the DMA channels and IRQ settings used by your hardware. Or, you can try to make the problem go away by changing the DMA and IRQ settings by trial-and-error.

When you install your soundcard, the installation program provided by the manufacturer will generally also install a set of software "device drivers" that are used to communicate between the soundcard and your computer. The soundcard manufacturer's installation program will also in general modify the "system.ini" file in your Window's directory, inserting some information about the name, characteristics, and DMA and IRQ settings for the card. It is important that your system.ini file contain the correct information in order for the soundcard to function properly. You can check what DMA and IRQ settings the soundcard thinks it is using by running the Control Panel, choosing Drivers, and clicking on the name of the driver associated with your soundcard. The settings should be the same as those that you set up the soundcard with. If they are not, you can either re-install the soundcard software, or manually edit the "system.ini" file. The method for doing this should be explained in the documentation provided by the soundcard manufacturer. Please note that TsunamiPro does NOT modify any of the important control files on your computer (such as autoexec.bat, config.sys, win.ini. or system.ini), or any other files, for that matter.

Similar problems in transferring data at high speeds can occur if your computer CPU is too slow, your disk is too slow, or if it is badly fragmented. You can improve the ability to access data from your hard disk at high speeds by using a commercial disk de-fragmenter, such as that provided by the "DiskDoctor" utility in the Norton Desktop, or by DOS version 6. By using a non-fragmented hard disk, TsunamiPro will be able to read and write data most efficiently, since disk files will be stored in adjacent clusters on the disk, and TsunamiPro will not have to search all over the disk for available storage sites. Similarly, disk-access is streamlined by setting up a permanent "swap file" when running Windows 3.x in "386 Enhanced Mode". We strongly recommend that you run the 16 bit version of TsunamiPro in this mode, as it allows Windows to use "virtual memory" (ie, to use the hard disk as a giant fake memory buffer to hold the memory-contents used by TsunamiPro and other concurrently running programs). This is the default mode when you start up Windows by simply typing "Win" at the DOS prompt. The procedure for setting up a permanent swap file is explained in detail in the Microsoft Windows User's Guide.

If whatever problem you are having persists, please make a note of any error messages and of the exact sequence of steps that led to the error before contacting us at (970) 389-2714.

# License Agreement and Warranty

The software described in this manual and in the TsunamiPro Help System is protected by copyright laws which apply internationally. It is illegal to copy this software for any purposes other than those stated below.  Installing and using TsunamiPro constitutes a legal agreement to adhere to the terms stated in this License Agreement. If you do not agree to the terms stated in this License Agreement, please return all program and reference materials promptly to the place of purchase for a full refund.

1. LICENSE. Black Diamond Sound Systems  licenses you to use a single copy of the supplied software on a single terminal. Using the software on a network or on more than one computer or terminal at a time is forbidden.

2. COPYRIGHT. All rights to the enclosed software are owned by Black Diamond Sound Systems and are protected by international copyright laws. You may make one copy of the software for backup purposes, and transfer it to a single hard disk, but making or distributing multiple copies of the software is illegal. You may transfer the software and documentation to a third party only if you retain no copies of it and the recipient agrees to the terms stated herein. It is illegal to disassemble, reverse engineer or otherwise obtain and use portions of the code constituting the enclosed software.

3. WARRANTY. Black Diamond Sound Systems warrants the supplied software and documentation to be free of defects in material and workmanship for a period of 30 days from date of purchase from an authorized Black Diamond Sound Systems distributor. Black Diamond Sound Systems will replace any defective software or documentation when the defective products are returned, along with proof of purchase, within the warranty period. Black Diamond Sound Systems liability is limited to the replacement of defective products, and Black Diamond Sound Systems will not be liable for typographical errors in the documentation, or for claims of damages based on inconvenience, loss of time, or commercial loss.

Microsoft, Windows, and MS-DOS are registered trademarks of the Microsoft Corporation.

# INDEX

3

386 Enhanced Mode, 94

A

Accelerando, 48

Action potentials, 8

Add echos, 48

Add Special Effects, 48

Adjust Frequency Components, 48

Adjusting The Display, 90

ADPCM File Compression, 12

Amplify Waveform, 48

Analog to digital converter, 8

Auditory system, 8, 9

Automatic Reformatting, 27

B

Building A Composite Waveform, 88

C

Change pitch, 48

Change Tempo, 48

Chase-lock Playback To External Signal, 32

Chorus effect, 48

Clear, 26

Closing A File, 10

Compress / expand dynamic range, 48

Contents, 86

Context-sensitive Help, 90

Cool Things You Can Do, 6

Copy, 26

Copying, 27

Copying A Single Channel, 27

Copying To a New File, 11

Create New File, 11

Creating A New Waveform, 11

Crop, 26

Cross Copying, 27

Cross Mixing, 27

Cross Pasting, 27

Cue list, 88

Customer Support, 6, 7

Cut, 26

Cutting, 27

D

Decibel, 9

Deleting A File, 10

Digital filters, 48

Display System Devices, 36

Drag And Drop, 27

Drag Drop, 11

E

Echo, 48

Edit Options, 26

Equalize, 48

Erase Clipboard Contents, 26

Exiting Program, 10

Exporting A File, 10

External Sampler, 32

F

File Compression, 12

File Menu Options, 10

Filter waveform, 48

Flanging, 48

Four-channel Parametric Equalizer, 48

G

Generate pure tones, 48

Get rid of clicks and pops, 36

Glissando, 48

Go to beginning of file, 36

Go to end of file, 36

Go to highlight, 36

H

Help, 90

Horizontal Magnification, 90

I

Importing A File, 10

Inner ear, 8

Insert, 26

Insert Silence, 48

Installation, 7, 94

L

Loop Playback, 32

M

Markers, 88

Mix, 26

Mixing, 27

Modulate waveform, 48

Multi-track List, 88

N

Noise reduction, 48

O

Opening A File, 10

Overwrite, 26

P

Pan Effect, 48

Paste, 26

Pasting, 27

Play Clipboard, 26

Play Continuously, 32

Play in reverse, 48

Playing A Waveform, 86

Preferences, 10

Program Overview, 5

R

Recording, 76

Redo, 27

Reduce noise, 48

Registration, 6

REplace, 26

Resolution, 9

Reverb, 48

Reverse, 48

Review And Cue, 90

Ritardando, 48

S

Sampling Rate, 8

Save, 12

Save As, 12

Save Highlighted Section, 12

Saving A Waveform, 12

Saving Files In Different Formats, 12

Scale waveform manually, 48

Select All, 26

Set Desired Number Of Times To Play  A Selection, 32

Setup Menu Options, 32

Setup Playback, 32

Silence, 48

Single Channel Copying, 27

Single-sample edditing, 36

Sinusoidal modulation, 48

Sound, 8

Special Effects, 48

Storage Requirements, 9

Synchronizing Recording Or Playback, 76

T

Tempo, 48

Time Bar, 90

Tremulo effect, 48

Trim, 26

Tympanic membrane, 8

U

Undo, 27

Update The Display, 36

Updates, 7

Updating The Display, 90

Use The CD Player, 36

Use the Midi Keyboard, 36

Use The Midi Player, 36

Use The Mixer, 36

User-drawn envelope, 48

Using Time Markers, 88

V

View Frequency Components Of Waveform, 36

View Magnified Image, 36

View Menu Options, 36

Visible Display Time, 90